Groundwater Watch List: Pharmaceuticals Pilot Study

Monitoring Data Collection and Initial Analysis

Final – January 2016

Written by Tony Marsland and Susie Roy (Amec Foster Wheeler)

EUROPEAN COMMISSION

Directorate-General for the Environment

Unit C1 - Water

Contact: Elisa Vargas Amelin

E-mail:	Elisa.VARGAS-AMELIN@ec.europa.eu

European Commission
B-1049 Brussels

Groundwater Watch List: Pharmaceuticals Pilot Study

Monitoring Data Collection and Initial Analysis

Final – January 2016

Issued by:

.......................

Tony Marsland (Amec Foster Wheeler Associate)

Approved by:

.......................

Mike Carey (Amec Foster Wheeler)

Amec Foster Wheeler Environment & Infrastructure UK Limited

Canon Court North,

Abbey Lawn, Abbey Foregate

Shrewsbury SY25DE

United Kingdom

Tel +44 (0)1743 342000

Doc Reg No.  36438rr007i2

Disclaimer

This document has been prepared for the European Commission however it reflects the views only of the authors, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

Any disclosure of this report to a third party is subject to this disclaimer.  The report was prepared by AMEC at the instruction of, and for use by, our client named on the front of the report.  It does not in any way constitute advice to any third party who is able to access it by any means.  AMEC excludes to the fullest extent lawfully permitted all liability whatsoever for any loss or damage howsoever arising from reliance on the contents of this report.  We do not however exclude our liability (if any) for personal injury or death resulting from our negligence, for fraud or any other matter in relation to which we cannot legally exclude liability.

January 2016

Europe Direct is a service to help you find answers to your questions about the European Union. Freephone number (*): 00 800 6 7 8 9 10 11 (*) The information given is free, as are most calls (though some operators, phone boxes or hotels may charge you).

LEGAL NOTICE

This document has been prepared for the European Commission however it reflects the views only of the authors, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

More information on the European Union is available on the Internet (http://www.europa.eu).

Luxembourg: Publications Office of the European Union, 2014

ISBN [number]

doi:[number]

© European Union, 2014

Reproduction is authorised provided the source is acknowledged.

Printed in [Country]

Printed on elemental chlorine-free bleached paper (ecf)

Printed on totally chlorine-free bleached paper (tcf)

Printed on recycled paper

Printed on process chlorine-free recycled paper (pcf)

Image(s) © [artist's name + image #], Year. Source: [Fotolia.com] (unless otherwise specified)

Table of Contents

Executive Summary	1

List of abbreviations	2

Chapter 1: Legislative and technical background	3

1.1	Introduction	3

1.2	Legislative drivers and Watch List concept	3

1.3	Context and scope of the Pilot Study and this report	4

2-	DATA COLLECTION	6

2.1	Data request	6

2.2	Datasets provided by Participating Countries	7

2.3	Data aggregation process	7

3-	Terminology and quality control issues	9

3.1	Interpretation of reported LoQs	9

3.2	Definition of pharmaceuticals and substances of concern	11

3.3	Variations in substance names	11

3.4	Formatting of tabulated data.	12

4-	DATA ASSESSMENT	13

4.1	Data sources and what they represent	13

4.2	Description of, and key to, tabulated data	13

5-	SUMMARY RESULTS	15

6-	CONCLUSIONS AND RECOMMENDATIONS	20

References	22

Annex 1 : Raw data	23

Annex 2 : Substance names by dataset and CAS numbers	23

Annex 3 : Aggregated data - Master Table	38

Annex 4 – Other Aggregated data	50

# Executive Summary

- Recital 4 of Directive 2014/80/EU (amending the Groundwater Directive,2006/118/EC - GWD) requests that a Watch List for groundwater pollutants be established under the Common Implementation Strategy for Water Framework Directive (2000/60/EC) to facilitate the identification of substances, including emerging pollutants, for which groundwater quality standards or threshold values should be set.  The derivation of a Watch List is the first step in gathering evidence to support any future decision to include new pollutants of European wide or national concern in Annex I or II of the GWD.  The process is intended to be as simple as possible and carried out on a voluntary basis, the aim being to continuously improve the knowledge base and monitoring programmes of MS.

Following discussion in CIS Groundwater Working Group (WGGW), it was agreed that a Pilot Study should be conducted on pharmaceutical chemicals, on the basis that this is a prominent group of emerging contaminants that has already been successfully monitored in EU Member and Associated States.  This report describes the monitoring data collection and initial analysis phase of the pilot study.

Following a request for data in 2015, by January 2016 twelve Member and Associated States had submitted datasets, which have been anonymised in this report by allocating an identifying number for each participating country (PC).  The data have been submitted and reported in Excel spreadsheets but are underpinned by an Access database which can be used to undertake additional assessment beyond that reported here.

Preliminary assessment of the raw data revealed a wide range in the reported Limits of Quantification (LoQ), raising questions over the consistency of reporting/interpretation of these values by analytical laboratories.

To facilitate accurate identification of substances, PCs were requested to submit CAS numbers alongside the substance name.  Less than half of the datasets received had CAS numbers but based on these returns and additional searches, a partial table of CAS nos. and substances has been prepared which also gives the “Common“ substance name in English and that given in the PC language, where this is different.

A very wide range of pharmaceutical substances (in total approximately 300 different substances) have been monitored by PCs and are identified in the aggregated data. However, there appear to be differences in view between PCs on what should be included in this category and some substances may be under-represented in the summary data.

The tabulated summary data is based on the number of sites where a substance has been detected above the LoQ and as such only gives an initial view of the extent of contamination by these substances and indirectly an indication of their persistence.  WGGW proposes to do further work to look at the potential risks to WFD objectives posed by individual substances by looking at exposure data and substance properties.

Tables are presented which compare LoQs for selected substances, rank the substances by the number of PCs which monitor them and number of sites monitored.  Further tables rank the substances by number of sites monitored and detection rate.

Carbamazepine is the substance most widely analysed by PCs, followed by Diclofenac.  Carbamazepine is also frequently detected above the LoQ (at 12% of sites), but Diclofenac has a relatively low detection rate (1%).  As a percentage of sites monitored, Paracetamol is the most frequently detected substance (24% of sites).  In addition to these substances, Diatrizoic Acid, Primidon, Ibuprofen and Clofibric Acid are prominent in the summary data.

The report identifies a number of issues that will need to be addressed going forward, such as:

- agreement on the definition and use of LoD and LoQ, with checks on the data submitted to date;

- completion of the list of CAS numbers and names;

- agreement on protocols for data confidentiality, access, and maintenance of any Watch List databases.

The Watch List concept paper that is being developed by WGGW will describe the overall assessment process, moving from the collection and assessment of monitoring data to the collection of substance specific properties and a ranking based on risk assessment.  The concept paper will provide a further opportunity to develop thinking on the issues raised in this report.  Details of Watch List process will be elaborated under the CIS Work Programme 2016 - 2018.

# List of abbreviations

CAS no. – Chemical Abstracts Service Registry number

CIS – Common Implementation Strategy for the Water Framework Directive

EC - European Commission

ECHA – European Chemicals Agency

GWB – Groundwater body

GWAAE – Groundwater Associated Aquatic Ecosystem

GWD – Groundwater Directive (2006/118/EC)

GWDTE – Groundwater Dependent Terrestrial Ecosystem

GWQS - Groundwater Quality Standard (GWD Annex I)

ID – Participating Country anonymised identification number

LoD – Limit of Detection

LoQ – Limit of Quantification

MS - Member States

PC – Participating Countries

SWB – Surface water body

TV – Threshold Value

WFD – Water Framework Directive (2000/60/EC)

WGGW – CIS Working Group on Groundwater

# Chapter 1: Legislative and technical background

## Introduction

As the largest body of freshwater in the European Union, groundwater is a valuable resource on which many ecosystems are dependent and is a major source for drinking water, irrigation and industrial purposes.  Though soil cover and overlying strata often protect groundwater from inputs of contaminants, once it is contaminated groundwater can be difficult and expensive to remediate.  The combination of a wide range of pressures arising from human activity and the generally long residence time of groundwater in the subsurface can result in long lasting contamination.  Groundwater policy and legislation in the EU therefore emphasise the need to prevent contamination and deterioration in groundwater quality.

Monitoring is a key element in groundwater management.  However, the costs of this monitoring often restricts the number of sampling points, range of determinands and sampling frequency in comparison with what is often achieved for surface water.

All of these factors are drivers both for focused surveillance of groundwater quality and sharing of data between Member States (MS), so that new or emerging contaminants are detected, their impact on groundwater and its dependent ecosystems and uses assessed and any necessary remedial/preventative measures are promptly implemented.

## Legislative drivers and Watch List concept

Article 3 of the Groundwater Directive (GWD - 2006/118/EC) describes the criteria for assessing the chemical status of a groundwater body, including the groundwater standards for nitrates and pesticides as noted in Annex I of that Directive and Threshold Values (TVs) established by MS in accordance with Annex II of the Directive.  TVs are set for individual pollutants that present a risk to a groundwater body and are a trigger for further assessment of the impact of that pollutant.  The GWD indicates that TVs should be focused on the protection of the groundwater body, including actual or potential legitimate uses or functions of groundwater and the interactions with associated surface waters (GWAAE) and directly dependent terrestrial ecosystems (GWDTE).

The pollutants currently listed in Annex I and II, as amended by Directive 2014/80/EU, represent what are currently considered to be the pollutants of most concern from a groundwater perspective at an EU level.  However, Recital 4 of the Directive 2014/80/EU states that:

- “The need to obtain and respond to new information on other substances posing a potential risk should be acknowledged. Therefore, a watch list for pollutants of groundwater should be established under the Common Implementation Strategy for Directive 2000/60/EC of the European Parliament and of the Council to increase the availability of monitoring data on substances posing a risk or potential risk to bodies of groundwater, and thereby facilitate the identification of substances, including emerging pollutants, for which groundwater quality standards or threshold values should be set.”

Thus, the derivation of a Watch List is the first step in gathering evidence to support any future decision to include new pollutants in Annex I or II of the GWD and is a tool to identify “new” substances of European wide or national concern. The process is intended to be as simple as possible and carried out on a voluntary basis, the aim being to continuously improve the knowledge base and monitoring programmes of MS.  Note that there are significant differences in purpose and legislative basis between the groundwater Watch List and the surface water Watch List, the latter being required under the Priority Substances Directive (Directive 2008/105/EC as amended by Directive 2013/39/EU).  The differences and links will be explored in more detail in the concept paper noted below.

## Context and scope of the Pilot Study and this report

The Pilot Study described in this report is the first step by the CIS Groundwater Working Group (WGGW) in addressing the need to prepare a Watch List.  A volunteer group started work on this topic during 2014 and discussions took place during WGGW plenary meetings in 2014 and 2015, with a special volunteer group meeting in Berne in September 2015.  Figure 1 summarises the overall process agreed by the group.

Figure 1: Watch List process overview.  Source: outcome of the Berne meeting – discussed and agreed upon by WG GW (Meeting in Luxembourg, Oct. 2015)

During the 2014 discussions, it was agreed that a Pilot Study should be conducted on pharmaceutical chemicals, on the basis that this is a prominent group of emerging contaminants that has already been successfully monitored in EU Member and Associated States.  This Pilot Study would represent “Column I” of the process as outlined above.

The preliminary results of this Pilot Study, with data from 6 MS and associated countries, were presented at the WGGW meeting in Brussels in April 2015.  Subsequently, further data have been obtained from 6 additional Member States and Amec Foster Wheeler has been asked to compile the results of the Pilot Study and prepare this report.

The study aims to collate the best available data and conduct an initial analysis; this will assist WGGW in gaining experience and developing optimised procedures for data collection and assessment that will be a solid base for future work on emerging contaminants in groundwater.

By agreement with WGGW the data provided in this report has been anonymised.

In parallel with the Pilot Study the volunteer group is preparing a Watch List concept paper which is due to be completed in mid-2016.  The concept paper will describe in more detail the purpose, policy and technical background to the groundwater Watch List and outline the determination process for candidate substances for the list. This process will include data collection, exposure assessment, risk assessment, prioritisation and data quality/knowledge gaps.  Finally the paper will describe how to implement the procedures, including the results of this pilot study and the lessons learnt.  Based on this, further data requests would be undertaken on groups of substances of concern, such as pesticides and their metabolites/breakdown products.

# DATA COLLECTION

## Data request

Following the presentation on the need for a Watch List to the Rome meeting of WGGW in October 2014 by Rüdiger Wolter (DE), a volunteer group was formed and the scope of the Pilot Study on pharmaceuticals was agreed.  MS were invited to submit data on the template presented at the meeting, as noted in Table 1.

Table 1: Sample of data return template (adapted from Rome presentation)

| Substance or metabolite | CAS | Total number of sites | Number of sites |  |  |  |  |  |  | Type of water | No. of MS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Maximum concentration at sites |  |  |  |  |  |  |  |  |
|  |  |  | below (LOQ) | > LOQ to 0.05 µg/l | > 0.05 to 0.1 µg/l | > 0.1 to 1.0 µg/l | > 1.0 to 3.0 µg/l | > 3.0 to 10.0 µg/l | > 10.0 µg/l |  |  |
| Carbamazepine |  | 962 | 919 | 27 | 9 | 7 | 0 | 0 | 0 | 1 | 1 |
| Diatrizoic Acid |  | 329 | 297 | 21 | 1 | 9 | 1 | 0 | 0 | 1 | 1 |
| 4-Formylaminoantipyrin |  | 129 | 122 | 4 | 3 | 0 | 0 | 0 | 0 | 1 | 1 |
| Clofibric acid |  | 687 | 677 | 1 | 3 | 6 | 0 | 0 | 0 | 1 | 1 |
| Bezafibrate |  | 635 | 633 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |
| Bisoprolol |  | 395 | 394 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |

Note: Type of water = 1 = untreated groundwater

It was emphasised that only existing, aggregated monitoring data were being requested, with data summarised at the end of the year.  Although all data were welcomed, it was noted that ideally the data should come from established regular or specific monitoring programmes (as opposed to local/short term site investigations), as well as data from local authorities and raw water data from water supply companies.  Data on the sites analysed and analytical methods were requested.  It was noted that participating countries (PC) were responsible for data quality and reliability.

Wherever possible, in addition to the chemical name of the substance, the unique Chemical Abstracts Service (CAS) registry number for that substance was requested, together with the limit(s) of quantification (LOQ) for that substance, but no definition for this latter term was given (see section 3.1).

Initially 8 countries responded, but only 6 (5 EU MS) were able to provide data on pharmaceuticals.  These data were summarised in a presentation to the following WGGW meeting in Brussels in April 2015 which is available on CIRCABC and summarised in Table 2 below.  At this stage, the data revealed that 171 different pharmaceuticals (human, veterinary, degradation products, X-ray contrast agents) had been monitored, of which 76 substances were analysed in two or more countries and 16 substances were analysed in four or more countries.

Subsequently, the addition of data from 6 more MS has allowed the dataset to be significantly expanded and updated results are presented in Chapter 5 and the Annexes to the report.  In all datasets the maximum concentrations have been reported.

Table 2:   Results from the preliminary data returns as presented in April 2015

It was assumed that countries used the maximum observed concentration if a site was analysed more than once a year.

For substance names in English please refer to Annex 2.

For updated table with additional data from more participating countries, see Table 6.

## Datasets provided by Participating Countries

Thirteen aggregated data tables were provided.  One MS provided two datasets from different administrations within the MS and an associated state contributed a dataset.  In the remainder of the report the 13 datasets are referred to as Participating Countries (PC) datasets for ease of reference.  Each PC has been given an identifying number (ID No.) to anonymise the data.

The PCs are AT, BE, CH, CZ, DE, DK, FR, IT, LU, NL, SI, and UK.

All the datasets provided were in Excel spreadsheet format, except one which was provided in Word document format.  One country provided four data sheets covering different periods of analysis, whilst other PC provided data for a single year, or a range of years in one tab.  The anonymised raw data are given in Annex 1 as an Excel file.

## Data aggregation process

The first stage of data processing involved the checking of each of the sheets to assess the format, in terms of ranges of data supplied, date ranges, number of substances, LoQ / LoD values, and any CAS numbers provided.  The format of the tables is key to aggregation and comparison of the whole dataset.  Differing table structures and headings meant that, before merging all the data into one “master table”, the individual tables had to be put in the same format.  This formatting was carried out in an Excel workbook.  All sheets were imported in their provided format.

A rapid comparison of table headers, number of substances reported and ranges provided was made.  This included a summary of all substances reported in the 13 collated datasets.  As different PC reported the same substance with varying spelling, a “common name” was identified, using web searches where necessary, including the European Chemicals Agency (ECHA) database.  Any duplicates in the list were removed in Excel, to create a list of 300 substances reported on.

A standard format was set up in one tab of the workbook, and this was used to import data in one format into an Access database. The “common name” table, including all variations of spelling of substances, with an allocated common name, was also imported to the database. As LoQ and CAS numbers were not reported by all PC, these data were excluded from the Access database tables.  Analysis of these data was made in the Excel spreadsheet.

All imported tables were then combined into one “master table” including all the results and a unique ID of PC + year(s) of reported samples.  This tabulated database was then exported back to the Excel spreadsheet.

The database was also used to recreate the tables produced and presented at the April 2015 Brussels meeting.  Queries based on the “common name” table and the “master table” were created, which produced tables of all reported substances, with the number of:

- PC reporting these substances;

- sites reported;

- detections reported; and

- detections above 0.1µg/l reported.

These tables have been provided in the Excel workbook, and compared to the tables produced for the Brussels meeting.

# Terminology and quality control issues

## Interpretation of reported LoQs

Limit of Quantification (LoQ) values have been provided by 5 PC and these are noted in Table 3 below for the four substances most often quoted by these PC.  Values were assumed to be provided as µg/l, although the values for one PC were given in ng/l and these have been converted to µg/l.  This comparison demonstrates significant (orders of magnitude) variability in reported LoQ range for individual substances, the reasons for which are unclear.  In particular, the PC10 values tend to be higher (although not exclusively).  The differences could be linked to confusion between LoD and LoQ reported by laboratories or to the difference in the analytical methods used for some substances (screening methods are less sensitive than targeted methods).

Table 3: LoQ values reported for selected pharmaceuticals (for 5 PCs)

| Substance | PC 4 | Year | PC 10 | Year | PC 11 | Year | PC 3 | Year | PC 5 | Year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carbamazepine | 0.0005 | 2008 | 0.01 | 2012 | <0.006 | 2014 | 0.0004 | 2014 |  |  |
|  |  |  | 0.01 | 2005/2006 |  |  |  |  |  |  |
|  |  |  | 0.01 | 2011-2014 |  |  |  |  |  |  |
|  |  |  | 0.01 | 2014 |  |  |  |  |  |  |
| Diclofenac | 0.0002 | 2008 | 0.01 | 2005/2006 | <0.012 | 2014 | 0.0022 | 2014 |  |  |
|  |  |  | 0.01 | 2011-2014 |  |  |  |  |  |  |
|  |  |  | 0.01 | 2014 |  |  |  |  |  |  |
| Ketoprofen | 0.001 | 2008 | 0.01 | 2012 | <0.005 | 2014 | 0.0033 | 2014 |  |  |
|  |  |  | 0.01 | 2011-2014 |  |  |  |  |  |  |
|  |  |  | 0.01 | 2014 |  |  |  |  |  |  |
| Sulfamethoxazole | 0.0005 | 2008 | 0.01 | 2012 | <0.006 | 2014 | 0.0006 | 2014 | 0.03 | 2006 |
|  |  |  | 0.02 | 2011-2014 |  |  |  |  |  |  |
|  |  |  | 0.01 | 2014 |  |  |  |  |  |  |

Note – all LoQ values are given in µg/l.

#### Use of Terms

A range of terms are used by laboratories to report the presence of a substance in water.  Most common are Limits of Detection (LoD) and Limits of Quantification (LoQ).  Definitions of these terms, taken from the QA/QC Directive, are given below.

LoD: the output signal or concentration value above which it can be affirmed, with a stated level of confidence that a sample is different from a blank sample containing no determinand of interest.

LoQ: a stated multiple of the limit of detection at a concentration of the determinand that can reasonably be determined with an acceptable level of accuracy and precision. The LoQ can be calculated using an appropriate standard or sample, and may be obtained from the lowest calibration point on the calibration curve, excluding the blank.

Note: The QA/QC Directive lays down technical specifications for chemical analysis and monitoring of water status in accordance with the WFD, and therefore these specifications are relevant to the Watch List work.

There are a range of other terms in use by laboratories which can cause confusion and in work undertaken on other projects AmecFW have encountered a surprising lack of consistency between laboratories on the understanding and use of these terms and the way results are quoted to clients.

#### Limits of Detection

For a given substance, laboratories can report different LoDs. This may reflect the method adopted by the lab, and in some cases may represent differences in laboratory equipment.  In developing a methodology for analysis, laboratories will try to develop a method with a LoD that is 10% of the concentration of interest (e.g. environmental standard).  However, whilst it may be feasible to achieve this, the method may not meet the criteria set by laboratory accreditation schemes and therefore the laboratory will report a higher LoD that is associated with an accredited method.

The LoD will also be a function of the matrix and therefore for contaminated samples, it may not be feasible to achieve a clean water LoD.  It has been noted that due to the variability in baseline quality of groundwater some laboratories may routinely treat groundwater samples as “dirty water” samples.

#### Limits of Quantification

In theory the LoQ will be higher than the LoD (typically 3 times the LoD).  In Europe, laboratories often provide both the LoD and LoQ, but sometimes the laboratory only reports the LoD or the LoQ.  In some cases the LoD may be equivalent to the LoQ, or may reflect the LoD for an accredited method.  A review of UK laboratories by AmecFW found that some laboratories treat LoD and LoQ as the same, and will only differentiate if requested.

It is clear that there is potential for confusion over the use of this terminology and also what the laboratory is reporting.  This may be the source of some of the variation noted in Table 3.  Given the number of laboratories involved, some variation in reported LoQs is to be expected but it is important that better consistency is achieved to improve comparability of results.  It is recommended that where data are used in Watch List assessments and where a specific check has not previously been conducted, the laboratory is requested to confirm their use of terms and method of reporting.

In future, the use of terms for LoD and LoQ as defined in the QA/QC Directive should contribute to harmonisation and common understanding.

## Definition of pharmaceuticals and substances of concern

Though no detailed definition of “pharmaceuticals” has been adopted by the volunteer group, we understand that this is taken to include human and veterinary medicines, their degradation products, and X-ray contrast agents.  Around 300 different substances are listed in the updated data from the 13 datasets noted above.  A screening process is to be developed by the volunteer group to identify substances of concern from this long list, which, based on the discussions held to date, is likely to consider factors, such as:

- the extent or potential extent of groundwater contamination;

- the significance of concentrations in relation to environmental quality standards (where these exist);

- chemical characteristics such as toxicity, persistence etc.;

- risk to WFD environmental objectives (including assessment of pathways);

- extent of usage of source chemicals (extent of existing pressures).

Further details can be seen in chapter 1.3, chart “Process to set up a Watch List” – Column II – “exposure assessment” and “Risk assessment”.

Based on the number of detections, the data as presented in this report give an initial view of the extent of contamination by some substances and indirectly some indication of their persistence.  No other assessment is feasible without additional data outside of the scope of the pilot study.  The next step of the Watch List process is to highlight those substances that have the potential to present a high risk of failing WFD objectives via a screening process as noted above.

## Variations in substance names

PC have used a variety of names and variations in spelling of substances and only a few have provided unique CAS (Chemical Abstracts Service) registry numbers.  Some PC have indicated that it was time consuming to look up and verify CAS numbers.  However, the absence of CAS numbers from some datasets and the variation in substance names resulted in uncertainty equating substances between datasets, with implications for the aggregation of data.

For clarity and future reference purposes, a complete list of all the names used for substances, broken down by dataset, is provided in Annex 2, with a “Common Name” in English for each of these name variations and, for the majority of substances, a CAS number.  Although AmecFW has undertaken extensive cross checks of the substance names and CAS numbers provided and has added substantially to the list of CAS numbers, there has not been sufficient time within the present project to provide a complete list of CAS numbers and confirm that these equate to the substances noted in the PC datasets.  Annex 2 is formatted such that confirmed CAS numbers are in bold and queries (inconsistent CAS numbers) are noted in italics (these will need to be referred back to the PC concerned, for checking with the laboratories who provided the analysis).

To ensure data combination and summaries are accurate, the use of agreed names and CAS numbers is strongly recommended and for future data collection exercises we consider it essential that PC datasets include CAS numbers, as the assignment of a substance name to a CAS number is best undertaken at source (that is, in conjunction with the laboratory undertaking the analysis).  However, it may be helpful for future data requests to provide a starting list of substances and CAS numbers with the request and then ask PC to confirm that the substance names they are using correspond to the description in the table and add to the table as necessary.

Although it has not been possible to completely populate the Annex 2 table with CAS numbers, it is recommended that this is undertaken prior to any further assessment work on pharmaceuticals as a quality control measure.  The database can then be upgraded to include CAS numbers so that statistical tables can be easily produced that include both names and CAS numbers.

## Formatting of tabulated data.

The following re-formatting of the tables submitted by PC has been necessary to ensure consistency and enable aggregation and reporting of the data:

- Some table headers have been modified or, where there were gaps, added;

- One PC provided an additional data range between 0.05 and 0.075 µg/l, but nothing was reported for pharmaceuticals in this column, so it was removed;

- Text and number formats have been amended (PC have provided data either for one year or for a range of years, and sometimes for both); and

- Some PC have used a hyphen in the LOQ to 0.05 µg/l column and this has been assumed to indicate a lack of analysis rather than no detection.

For future data requests the following measures are recommended to improve consistency and aid subsequent analysis of the data:

- Separate cells are used for start and end dates for monitoring data;

- Column and text/numeric formats should be standardised, with a view to easing transfer of the data into an Access database;

- All substances should have a CAS number; and

- Notes on completion of the tables and a worked example should be provided.

# DATA ASSESSMENT

## Data sources and what they represent

As noted in Chapter 2, the original request to potential contributors advised that whilst all data was welcomed, data from normal or specific monitoring programmes was of primary interest (in contrast to localised/short term investigations).  Data submissions should indicate where possible whether the data were representative of widespread or localised pollution.  If data reflected the latter this should be flagged.  PC3 has indicated that their data are from focused investigations and have acknowledged that this will not be representative of widespread problems.  These data have been included within this report and the database.

Several other potential sources of bias may be present in the data generally, such as:

- Where the substance data are generated from a small proportion of PC (for example, only PC10 has submitted analyses for EDTA (Ethylenediaminetetraacetic acid), but there are numerous results from this PC);

- Where a substantially different LoQ is used by one PC for a substance;

- Where inputs of a substance and therefore detections are spatially restricted (for example, some substances may have very restricted uses but may be frequently detected where they are used.  Despite representing a relatively high risk to groundwater quality they may be positioned well down the overall ranking tables in terms of occurrence).

Methods for identifying and dealing with such bias will need to be developed but are not within the scope of this report.

## Description of, and key to, tabulated data

Table 4 below provides a key to all the data presented in this report and submitted by PC.  The Annexes to this report contain the complete datasets in Excel format as electronic file attachments (this enables the tables to be sorted using different criteria, if necessary).  Summary data or partial datasets are presented in the main text and as printed tables in the Annexes.  The main summary aggregated data are presented in Tables 6, 7 and 8 and follow a similar format to those previously presented to WGGW meetings in 2015 for the initial dataset from 6 PC. The aggregated data have been derived from an Access database into which all the raw data have been imported – this is not attached to this report as the data are attributed in the database to individual PC.

Note that PC13 has supplied data for some substances that have been detected in laboratory scans but which have not been quantified – these have not been used in the statistical tables (6, 7 & 8) but are included in Annexes 1 and 2, are noted at the end of Annex 3 where their detection is unique to the PC13 dataset and included in full as a separate file in Annex 4.

Table 4: Key to data tables

| Item | Description | Contents | Format |
| --- | --- | --- | --- |
| Annex 1 | Anonymised PC data returns | Raw detection data, including LOQ and CAS nos. provided by PC | Excel Workbook |
| Annex 2 | Substance names by PC dataset, with CAS numbers | Aggregation of different names used by PC, a “common name” and CAS number. | Excel Workbook and hard copy |
| Annex 3 | Aggregated data – Master Table | Aggregation of Annex 1 detection data, ranked by total number of detections. | Excel Workbook and hard copy |
| Annex 4 | Other aggregated data | Full LOQ data submitted by PC; Full substance analysis and detection tables (see Tables 6 & 7). PC13 scan data (unquantified detects). | Excel Workbook |
| Table 1 (s2.1) | Data collection template | Template as sent to PC | Text |
| Table 2 (s2.1) | April 2015 preliminary results | Aggregation of 6 PC data presented to WGGW meeting, with no. of sites and detection rates by substance. | Text |
| Table 3 (s3.1) | LOQ values for selected substances | LOQs for four substances most commonly represented in PC datasets | Text |
| Table 5 (Ch5) | Number of substances in PC datasets | Total number of substances reported by each PC | Text |
| Table 6 (Ch5) | Substances detected in groundwater | Substances ranked by number of PC that monitor them and number of sites monitored – where 4 or more PC have monitored for that substance. | Text (full dataset in Annex 4) |
| Table 7 (Ch5) | Substances detected in groundwater | Substances detected by <4 countries but >5% detection rate, ranked by detection rate. | Text (full dataset in Annex 4) |
| Table 8 (Ch5) | Substances reported at > LOQ and > 0.1 µg/l | Substances ranked by detection > 0.1 µg/l | Text (full dataset in Annex 4) |

# SUMMARY RESULTS

As noted in Table 5, the number of substances monitored by PC and represented in the datasets varies widely from 4 to 135 substances.  In total approximately 300 different substances have been monitored by PC and a full list, using the Common Name is provided in Annex 2.

Note: for the purposes of this Pilot Study report we have assumed that all substances included in PC submissions are pharmaceuticals; in the time available and without precise definitions there was no rationale for excluding any of these substances.  Where queries regarding the inclusion of a substance have been raised this is noted in Annex 2.

Table 5: Number of substances in PC datasets

| PC ID | Number of Substances Reported | Time period for datasets |
| --- | --- | --- |
| 1 | 19 | 2006 and 2008 |
| 2 | 94 | 2009/2011, 2009 |
| 3 | 34 | 2014 |
| 4 | 9 | 2008 |
| 5 | 33 | 2006 |
| 6 | 135 | 2011 |
| 7 | 109 | 2013 |
| 8 | 37 | 2010-11 |
| 9 | 4 | 2013 |
| 10 | 135 | 2005/6,2012, 2011-14, 2014 |
| 11 | 19 | 2014 |
| 12 | 36 | 2013 |
| 13 | 54 | 2011/2014 |

Table 6 below gives a summary of the number of sites at which a substance has been analysed by four or more PC; this cut-off point is only used for illustrative purposes in this report; the full list of all substances is given in the electronic file in Annex 4.  Note that in Tables 6,7 & 8 the substances in bold italics are on the current surface water Watch List.

Carbamazepine is the substance most widely analysed by PCs, followed by Diclofenac.  In contrast, whilst Carbamazepine is also frequently detected above the LoQ (at 12% of sites), Diclofenac has a relatively low detection rate (1%).  As a percentage of sites monitored, Paracetamol is the most frequently detected substance (24% of sites) in Table 7.  Carbamazepine and Paracetamol stand out from the other data in this Table, which only considers detections where there are 4 or more PC.  Only three other substances in this table reach the 5% threshold, these being Sulfamethoxazole, Diatrizoic acid and Primidon.

Table 6: Substances detected in groundwater (4 or more countries)

| Common Name | Number of PC | Total no. of sites | >LoQ | >LoQ in % |
| --- | --- | --- | --- | --- |
| Carbamazepine | 12 | 3692 | 445 | 12% |
| Sulfamethoxazole | 12 | 2153 | 105 | 5% |
| Ketoprofen | 10 | 1275 | 8 | 1% |
| Diclofenac | 10 | 3060 | 34 | 1% |
| Sulfadimidine | 9 | 1314 | 19 | 1% |
| Sulfadiazine | 9 | 1265 | 9 | 1% |
| Trimethoprim | 9 | 1352 | 1 | 0% |
| Ibuprofen | 9 | 1865 | 49 | 3% |
| Paracetamol (Acetominophen) | 9 | 1036 | 248 | 24% |
| Naproxen | 8 | 1374 | 1 | 0% |
| Erythromycin | 8 | 1255 | 25 | 2% |
| Gemfibrozil | 8 | 1085 | 2 | 0% |
| Atenolol | 7 | 1499 | 10 | 1% |
| Bezafibrate | 7 | 884 | 3 | 0% |
| Oxytetracycline | 7 | 903 | 0 | 0% |
| Metoprolol | 7 | 1708 | 3 | 0% |
| Clofibric acid | 7 | 2294 | 46 | 2% |
| Clarithromycin | 7 | 1216 | 10 | 1% |
| Tetracycline | 6 | 931 | 0 | 0% |
| Enrofloxacin | 6 | 858 | 2 | 0% |
| propranolol | 6 | 1177 | 1 | 0% |
| Ciprofloxacin | 6 | 880 | 6 | 1% |
| Sotalol | 6 | 1479 | 25 | 2% |
| Diazepam | 6 | 879 | 9 | 1% |
| Fenofibrate | 5 | 746 | 1 | 0% |
| Diatrizoic Acid | 5 | 1394 | 66 | 5% |
| Ofloxacin | 5 | 731 | 3 | 0% |
| Lincomycin | 5 | 867 | 3 | 0% |
| Estrone | 5 | 818 | 1 | 0% |
| Ethinyl estradiol | 5 | 793 | 5 | 1% |
| Cyclophosphamide | 5 | 958 | 0 | 0% |
| Roxithromycin | 4 | 338 | 0 | 0% |
| Metronidazole | 4 | 676 | 16 | 2% |
| Chlortetracycline | 4 | 694 | 2 | 0% |
| Ranitidine | 4 | 728 | 2 | 0% |
| Norfloxacin | 4 | 676 | 3 | 0% |
| Codeine | 4 | 666 | 15 | 2% |
| Primidon | 4 | 628 | 31 | 5% |
| Betaxolol | 4 | 341 | 0 | 0% |
| iopromide | 4 | 781 | 1 | 0% |
| Salbutamol | 4 | 478 | 0 | 0% |
| Lidocaine | 4 | 540 | 11 | 2% |
| Phenazone | 4 | 1477 | 40 | 3% |
| Iomeprol | 4 | 757 | 1 | 0% |
| Iopamidol | 4 | 1462 | 37 | 3% |
| Indomethacine | 4 | 553 | 1 | 0% |
| Tylosin | 4 | 649 | 0 | 0% |
| Amoxicillin | 4 | 701 | 0 | 0% |
| Furosemide | 4 | 699 | 6 | 1% |
| Sulfathiazole | 4 | 676 | 0 | 0% |
| Venlafaxine | 4 | 169 | 1 | 1% |
| Fenofibric acid | 4 | 601 | 14 | 2% |
| Sulfadoxin | 4 | 90 | 0 | 0% |
| Sulfadimethoxine | 4 | 702 | 0 | 0% |
| Estradiol | 4 | 683 | 0 | 0% |
| Sulfamerazine | 4 | 215 | 2 | 1% |
| Florfenicol | 4 | 533 | 1 | 0% |

In Table 7 we have also listed those substances with a 5% or greater detection rate but where these are found in 3 or less PC datasets.  Whilst there is an increasing risk of bias and the data may be less representative with fewer datasets, the results give grounds for other PC to consider inclusion of these relatively frequently detected substances in their monitoring programmes, subject to a risk assessment.

Table 7: Substances detected in groundwater (<4 countries but >5% detection rate and > 2 sites)

| Common Name | Number of PC | Total no. of sites | >LoQ | >LoQ in % |
| --- | --- | --- | --- | --- |
| Oxazepam | 3 | 525 | 29 | 6% |
| Tramadol | 3 | 675 | 54 | 8% |
| Metformin | 3 | 787 | 56 | 7% |
| Gabapentin | 2 | 167 | 19 | 11% |
| Salicylic acid | 2 | 8 | 5 | 63% |
| 4-Formylaminoantipyrin | 2 | 137 | 7 | 5% |
| caffeine | 2 | 490 | 252 | 51% |
| Dehydro- erythromycin | 1 | 55 | 7 | 13% |
| Amidotrizoate | 1 | 24 | 5 | 21% |
| bisphenol-A | 1 | 105 | 12 | 11% |
| Crotamiton | 1 | 27 | 27 | 100% |
| Butabarbital | 1 | 6 | 6 | 100% |
| Clopidol | 1 | 11 | 11 | 100% |
| Pentobarbital | 1 | 26 | 26 | 100% |
| prozac | 1 | 5 | 1 | 20% |
| Sulfanilamide | 1 | 5 | 5 | 100% |
| Ethylenediaminetetraacetic acid (EDTA) | 1 | 105 | 61 | 58% |
| Epoxycarbamazepine | 1 | 32 | 6 | 19% |
| Methaqualone | 1 | 3 | 3 | 100% |
| Lamotrigine | 1 | 3 | 3 | 100% |

Finally, in Table 8 we have listed the aggregated data for the substances which are both detected at the LoQ level and the >0.1 µg/l level, ranked by this latter criterion; cases where the detections are from less than 4 PC datasets are shaded.  Taking out the shaded rows, Carbamazepine, Diatrizoic Acid, Diclofenac, and Primidon are again prominent, together with Ibuprofen, Paracetamol and Clofibric acid.

Table 8: Substances reported at > LOQ and > 0.1 µg/l

| Substance | No. of PC | Total number of sites | >LOQ | >0.1 µg/l |
| --- | --- | --- | --- | --- |
| Ethylenediaminetetraacetic acid (EDTA) | 1 | 105 | 61 | 61 |
| caffeine | 2 | 490 | 252 | 59 |
| Carbamazepine | 12 | 3692 | 445 | 57 |
| Ibuprofen | 9 | 1865 | 49 | 15 |
| Diatrizoic Acid | 5 | 1394 | 66 | 15 |
| Diclofenac | 10 | 3060 | 34 | 13 |
| Gabapentin | 2 | 167 | 19 | 13 |
| Metformin | 3 | 787 | 56 | 12 |
| Paracetamol (Acetominophen) | 9 | 1036 | 248 | 11 |
| Clofibric acid | 7 | 2294 | 46 | 10 |
| Primidon | 4 | 628 | 31 | 10 |
| Pentoxifylline | 3 | 859 | 9 | 9 |
| Acetylsalicylic acid | 3 | 600 | 7 | 7 |
| Phenazone | 4 | 1477 | 40 | 5 |
| Sulfamethoxazole | 12 | 2153 | 105 | 4 |
| Ioxithalamic acid | 3 | 329 | 12 | 4 |
| Erythromycin | 8 | 1255 | 25 | 3 |
| Iopamidol | 4 | 1462 | 37 | 3 |
| Ketoprofen | 10 | 1275 | 8 | 2 |
| Sulfadiazine | 9 | 1265 | 9 | 2 |
| Metronidazole | 4 | 676 | 16 | 2 |
| Lidocaine | 4 | 540 | 11 | 2 |
| Fenofibric acid | 4 | 601 | 14 | 2 |
| Propyphenazon | 3 | 954 | 11 | 2 |
| Benzotriazole | 2 | 115 | 4 | 2 |
| Oxolinic acid | 2 | 641 | 7 | 2 |
| Amidotrizoate | 1 | 24 | 5 | 2 |
| Trimethoprim | 9 | 1352 | 1 | 1 |
| Furosemide | 4 | 699 | 6 | 1 |
| Ivermectine | 3 | 726 | 1 | 1 |
| Tramadol | 3 | 675 | 54 | 1 |
| Dimetridazole | 2 | 150 | 1 | 1 |
| Hydrochlorothiazide | 2 | 547 | 12 | 1 |
| Oxcarbazepine | 2 | 426 | 6 | 1 |
| Progesterone | 2 | 524 | 3 | 1 |
| Diethylenetriaminepentaacetic acid (DTPA) | 1 | 105 | 1 | 1 |
| bisphenol-A | 1 | 105 | 12 | 1 |
| N-Acetyl-4-aminoantipyrin | 1 | 129 | 5 | 1 |

# CONCLUSIONS AND RECOMMENDATIONS

- As a pilot study, this exercise has brought together a substantial body of data in a relatively short time frame, illustrating that a very wide range of pharmaceuticals (~300 in total) have been monitored and detected in groundwater by the European countries participating in this study.

- The data have been captured in both an Access database and a series of Excel worksheets which should enable the data to be supplemented and subject to further screening and assessment, following the methodologies that are being developed by the WGGW Watch List volunteer group.

- The data presented here are a sample of the data available at a point in time.  The Watch List process will need to address how and when data are updated and, for the wider study, the potential workload in capturing, updating, processing, storing and assessing data for a potentially lengthy list of substances.  Consideration should be given to how screening processes might be used to optimise this workload, whilst focusing attention on substances with the potential to have a significant impact on WFD environmental objectives (e.g. substances noted from this study could include diclofenac, estrone, clarithromycin and erythromycin – which are all on the current surface water Watch List and are listed in Tables 6, 7 or 8).

- Some improvements to the data capture process should be considered for any future data requests:

- A clear definition of the substance group under consideration;

- Guidance notes and a worked example to accompany the data request template;

- Provision of separate start and end date columns for the submitted dataset;

- Consider whether both maximum and median data (including frequencies) should be submitted;

- Fixed formatting of text and numeric fields in the data template;

- Adaptation of the concentration/detection fields in the data template to the substance group of concern (smaller groups of substances may need to be identified);

- All substances submitted for inclusion should be accompanied by CAS numbers;

- An initial reference list of substances with unique CAS numbers provided with the data request, which PC can refer and add to if necessary.

- Some issues merit further attention prior to any further data capture and/or assessment:

- Agreement on the definitions of LoD and LoQ (as noted in the QA/QC Directive) and how these should be used in the assessment process;

- A check by PC of the data submitted to confirm that the LoQ has been used or not.  This may result in the need to update the database and tabulated summary data;

- The Annex 2 list of substances should be completed so that a unique CAS number is allocated to each Common Name with equivalent names in PC languages;

- The following on-going issues will also need to be addressed:

- How any databases created as part of the Watch List exercise should be maintained and who should be responsible for this;

- Written protocols over data confidentiality and access.

Future Steps

It is anticipated that this Pilot Study is the first of several data acquisition exercises to produce evidence to select candidate substances for the groundwater Watch List.  Other substance groups which may be considered include pesticides/biocides, perfluorinated compounds, and industrial chemicals.  The collated monitoring data will be combined with other technical data and put through an assessment process which may in due course produce recommendations for additions to the substances listed in Annexes 1 and 2 of the GWD.

This report has highlighted a number of issues that need to be addressed before any further data requests take place, some of which have been discussed at the meeting in Berne in October 2015.

The Watch List concept paper that is being developed by WGGW will describe the overall assessment process, moving from the collection and assessment of monitoring data to the collection of substance specific properties and a ranking based on risk assessment.  The concept paper will provide a further opportunity to develop thinking on the issues raised in this report, which may benefit from lessons learnt from the surface water Watch List process and comparisons between the results of these processes (whilst taking note of the differences in objectives).

Details of Watch List process will be elaborated under the CIS Work Programme 2016 - 2018.

# References

Directives

2000/60/EC establishing a framework for Community action in the field of water policy.  (Water Framework Directive).

2006/118/EC on the protection of groundwater against pollution and deterioration. (Groundwater Directive)

2008/105/EC on environmental quality standards in the field of water policy, amending and subsequently repealing Council Directives 82/176/EEC, 83/513/EEC, 84/156/EEC, 84/491/EEC, 86/280/EEC and amending Directive 2000/60/EC of the European Parliament and of the Council.  (Priority Substances Directive)

2009/90/EC laying down, pursuant to Directive 2000/60/EC of the European Parliament and of the Council, technical specifications for chemical analysis and monitoring of water status. (QA/QC Directive).

2013/39/EU amending Directives 2000/60/EC and 2008/105/EC as regards priority substances in the field of water policy

2014/80/EU amending Annex II to Directive 2006/118/EC of the European Parliament and of the Council on the protection of groundwater against pollution and deterioration.

Others

European Chemicals Agency (ECHA) database - http://echa.europa.eu

Quality Assurance Associates, 1999: “Understanding Laboratory Reporting Limits” Environmental Protection Magazine, 10, 37-41

# Annex 1: Raw data

# Annex 2: Substance names by dataset and CAS numbers

(See next page for hard copy of file)

Key to tables

| Notes : | CAS Nos. in bold - numbers agree in 2 or more PC and/or checked in ECHA database |  |  |
| --- | --- | --- | --- |
|  | CAS Nos. in shaded italics - numbers do not agree between datasets or in ECHA database |  |  |

| ANNEX 2 – SUBSTANCE NAMES, DATASETS AND CAS NUMBERS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PC and dataset | Substance or metabolite (PC description) | Common Name | CAS Number | Comment |
| 6(2011) | 14-Hydroxyclarithromycin | 14-Hydroxyclarithromycin | 81103-11-9 | Same CAS as Clarithromycin |
| 7(2013); 6(2011) | 17-alpha-Estradiol | 17-alpha-Estradiol | 57-91-0 |  |
| 10(2012) | oestradiol |  |  |  |
| 6(2011); 8(2010–11) | Estradiol | Estradiol | 50-28-2 |  |
| 2(2011); 7(2013) | 17-beta-Estradiol |  |  |  |
| 7(2013); 2(2011) | 17-alpha-Ethinylestradiol | Ethinyl estradiol | 57-63-6 |  |
| 10(2012) | 17α-ethinyloestradiol |  |  |  |
| 8(2010–11); | Ethinyl estradiol |  |  |  |
| 6(2011); 10(2005/2006) | Ethynyl estradiol |  |  |  |
| 13(2011-14) | 10,11-Dihydroxycarbazepine | 10,11-Dihydroxycarbazepine | 35079-97-1 |  |
| 6(2011) | 1-Hydroxy Ibuprofen | 1-Hydroxy Ibuprofen | 53949-53-4 |  |
| 5(2006) | 2-hydroxy-ibuprofen | 2-hydroxy-ibuprofen |  |  |
| 12(2013) | 4-Acetamidoantipyrin | 4-Acetamidoantipyrin |  |  |
| 2(2011) | 4-acetylaminoantipyrine | 4-acetylaminoantipyrine | 83-15-8 |  |
| 6(2011) | 4-Chlorobenzoic acid | 4-Chlorobenzoic acid | 74-11-3 |  |
| 6(2011); 7(2013) | 4-epi-Chlortetracyclin | 4-epi-Chlortetracyclin |  |  |
| 7(2013) | 4-epi-Oxytetracyclin | 4-epi-Oxytetracyclin |  |  |
| 7(2013) | 4-epi-Tetracyclin | 4-epi-Tetracyclin |  |  |
| 2(2011); 7(2013) | 4-Formylaminoantipyrin | 4-Formylaminoantipyrin | 1672-58-8 |  |
| 7(2013) | 4-Hydroxy-Sulfadiazin | 4-Hydroxy-Sulfadiazin |  |  |
| 7(2013) | Acetbutolol | Acetbutolol |  |  |
| 5(2006) | acetylsalicylsyre | Acetylsalicylic acid | 50-78-2 |  |
| 10(2012) | Acetylsalicylzuur |  |  |  |
| 6(2011) | Acide salicylique |  |  |  |
| 1(2008) | Acetyl-Sulfadimidin | Acetyl-Sulfadimidine |  |  |
| 12(2013) | Acetyl-sulfamethoxazol | Acetyl-sulfamethoxazole | 21312-10-7 | see N-Acetyl-Sulfamethoxazole |
| 10(2011-2014) | Acetylsulfamethoxazole |  |  |  |
| 2(2011) | N-Acetyl-Sulfamethoxazole | N-Acetyl-Sulfamethoxazole | 21312-10-7 | see Acetyl-sulfamethoxazole |
| 3(2014); 6(2011) | Acide fenofibrique | Fenofibric Acid | 26129-32-8 |  |
| 6(2011) | Albendazole | Albendazole | 54965-21-8 |  |
| 6(2011) | Alprazolam | Alprazolam | 28981-97-7 |  |
| 6(2011) | Altrenogest | Altrenogest | 850-52-2 |  |
| 7(2013) | Amidotrizoate | Amidotrizoate |  |  |
| 7(2013) | Amidotrizoelsäure | Diatrizoic Acid | 117-96-4 |  |
| 12(2013) | Amidotrizoesaeure |  |  |  |
| 2(2011) | Amidotrizoic acid |  |  |  |
| 10(2012) | amidotrizoïnezuur |  |  |  |
| 6(2011) | Acide diatrizoique |  |  |  |
| 13(2011-14) | Amino-mebendazole | Amino-mebendazole | 52329-60-9 |  |
| 6(2011) | Amiodarone | Amiodarone | 1951-25-3 |  |
| 13(2011-14) | Amitriptyline | Amitriptyline | 50-48-6 |  |
| 5(2006) | amlodipin | Amlodipine | 88150-42-9 |  |
| 6(2011) | Amlodipine |  |  |  |
| 2(2011); 10(2011-2014) | Amoxicillin | Amoxicillin | 26787-78-0 |  |
| 8(2010–11); 6(2011) | Amoxicilline |  |  |  |
| 5(2006) | ampicillin | Ampicillin | 69-53-4 |  |
| 6(2011) | Ampicilline |  |  |  |
| 6(2011) | Amprolium | Amprolium | 137-88-2 |  |
| 6(2011) | Androstenedione | Androstenedione | 63-05-8 |  |
| 13(2011-14) | Androstadienedione | Androstadienedione | 897-06-3 |  |
| 6(2011) | Androsterone | Androsterone | 53-41-8 |  |
| 2(2011) | Anhydro erythromycin | Anhydro erythromycin | 23893-13-2 |  |
| 2(2011); 3(2014); 6(2011); 7(2013); 8(2010-11); 10(2011-2014); 10(2012); 10(2014); 12(2013); 13(2011-14) | Atenolol | Atenolol | 29122-68-7 |  |
| 8(2010–11) | Atorvastatin | Atorvastatin | 134523-00-5 |  |
| 10(2011-2014): 12(2013) | Azithromycin | Azithromycin | 83905-01-5 |  |
| 10(2012) | azitromycine |  |  |  |
| 7(2013) | Be(n)zafibrat | Be(n)zafibrat |  |  |
| 5(2006) | bendroflumethiazid | bendroflumethiazid |  |  |
| 2(2009-2011); 10(2012) | Benzotriazole | Benzotriazole | 95-14-7 |  |
| 5(2006) | benzylpenicillin | benzylpenicillin |  |  |
| 7(2013) | beta-Sitosterol | beta-Sitosterol |  |  |
| 2(2011); 7(2013); 10(2011-2014); 11(2014) | Betaxolol | Betaxolol | 63659-18-7 |  |
| 10(2012); 10(2005/2006) | bezafibraat | Bezafibrate | 41859-67-0 |  |
| 12(2013) | Bezafibrat |  |  |  |
| 2(2011); 4(2008); 6(2011); 8(2010–11); 10(2011-2014); 11(2014) | Bezafibrate |  |  |  |
| 10(2012) | bisfenol-A | bisphenol-A | 80-05-7 | ? pharmaceutical |
| 2(2011); 7(2013); 10(2005/2006); 10(2012) | Bisoprolol | Bisoprolol | 66722-44-9 | see Bisoprolol-A |
| 10(2011-2014) | Bisoprolol-A | Bisoprolol-A | 66722-44-9 | see Bisoprolol |
| 6(2011); 13(2011-14) | Bromazepam | Bromazepam | 1812-30-2 |  |
| 7(2013) | Bromhexin | Bromhexin |  |  |
| 6(2011) | Buflomedil | Buflomedil | 55837-25-7 |  |
| 13(2011-14) | Butabarbital | Butabarbital | 125-40-6 |  |
| 6(2011) | Butalbital | Butalbital | 77-26-9 |  |
| 10(2012); 10(2014); 13(2011-14) | caffeine | caffeine | 58-08-2 |  |
| 10(2011-2014) | Coffeïne |  |  |  |
| 10(2011-2014) | Capecitabine | Capecitabine | 154361-50-9 |  |
| 7(2013) | Carazolol | Carazolol |  |  |
| 1(2006); 7(2013); 12(2013) | Carbamazepin | Carbamazepine | 298-46-4 |  |
| 2(2009-2011); 3(2014); 4(2008); 6(2011); 8(2010–11); 9(2013); 10(2011-2014); 10(2012); 10(2014); 10(2005/2006); 11(2014); 13(2011-14) | Carbamazepine |  |  |  |
| 6(2011) | Carbamazepine epoxide | Carbamazepine epoxide | 36507-30-9 |  |
| 6(2011) | Carboxyibuprofen | Carboxyibuprofen | 15935-54-3 |  |
| 6(2011); 10(2011-2014) | Cefazoline | Cefazoline | 25953-19-9 |  |
| 10(2011-2014) | Cefotaxim | Cefotaxim | 64485-93-4 |  |
| 10(2011-2014) | Cefuroxime | Cefuroxime | 55268-75-2 |  |
| 13(2011-14) | Celiprolol | Celiprolol | 56980-93-9 | PC gave CAS as 55268-75-2 |
| 10(2012) | chlooramfenicol | Chloramphenicol | 56-75-7 |  |
| 2(2011) | Chloroamphenicol |  |  |  |
| 10(2005/2006); 10(2011-2014) | Chloramphenicol |  |  |  |
| 6(2011) | Chlormadinone | Chlormadinone | 1961-77-9 |  |
| 2(2011); 10(2011-2014) | Chlorotetracycline | Chlorotetracycline | 64-72-2, 57-62-5 | PC10 CAS or name incorrect |
| 7(2013) | Chlortetracyclin | Chlortetracycline | 57-62-5 |  |
| 6(2011) | Chlortetracycline |  |  |  |
| 5(2006) | chlortetracyklin |  |  |  |
| 1(2008) | Chlortetrazyklin |  |  |  |
| 5(2006) | cimetidin | cimetidine | 51481-61-9 |  |
| 1(2008); 2(2011); 7(2013); 8(2010–11); 10(2011-2014) | Ciprofloxacin | Ciprofloxacin | 85721-33-1 |  |
| 6(2011) | Ciprofloxacine |  |  |  |
| 13(2011-14) | Citalopram | Citalopram | 59729-33-8 |  |
| 1(2008); 2(2011); 7(2013); 8(2010–11); 10(2011-2014); 12(2013) | Clarithromycin | Clarithromycin | 81103-11-9 | see 14-Hydroxyclarithromycin |
| 6(2011); | Clarithromycine |  |  |  |
| 10(2012) | claritromycine |  |  |  |
| 2(2011); 7(2013); 10(2011-2014) | Clenbuterol | Clenbuterol | 37148-27-9 |  |
| 12(2013) | Clindamycin | Clindamycin | 18323-44-9 |  |
| 10(2011-2014) | Clofibrate | Clofibrate | 637-07-0 |  |
| 2(2011); 8(2010–11); 10(2011-2014) | Clofibric acid | Clofibric acid | 882-09-7 |  |
| 10(2005/2006) | clofibrinezuur |  |  |  |
| 12(2013) | Clofibrinsaeure |  |  |  |
| 7(2013) | Clofibrinsäure |  |  |  |
| 5(2006) | clofibrinsyre |  |  |  |
| 6(2011) | Acide clofibrique |  |  |  |
| 13(2011-14) | Clonazepam | Clonazepam | 1622-61-3 |  |
| 13(2011-14) | Clopidol | Clopidol | 2971-90-6 |  |
| 6(2011) | Clorsulone | Clorsulone | 60200-06-8 |  |
| 6(2011); 7(2013) | Clotrimazole | Clotrimazole | 23593-75-1 |  |
| 2(2011); 10(2011-2014) | Cloxacillin | Cloxacillin | 61-72-3 |  |
| 6(2011) | Cloxacilline | Cloxacillin | 7081-44-9 | PC name or CAS incorrect |
| 6(2011); 13(2011-14) | Cocaine | Cocaine | 50-36-2 |  |
| 7(2013) | Codein | Codeine | 76-57-3 |  |
| 6(2011); 11(2014) | Codeine |  |  |  |
| 10(2011-2014) | Codeïne |  |  |  |
| 13(2011-14) | Cotinine | Cotinine | 486-56-6 |  |
| 13(2011-14) | Crotamiton | Crotamiton | 483-63-6 |  |
| 6(2011) | Cyamemazine | Cyamemazine | 3546-03-0 |  |
| 7(2013) | Cyclophosphamid | Cyclophosphamide | 50-18-0 |  |
| 2(2011); 6(2011); 8(2010–11); 10(2011-2014) | Cyclophosphamide |  |  |  |
| 6(2011) | Cytarabine | Cytarabine | 147-94-4 |  |
| 1(2008) | Danofloxacin | Danofloxacin | 112398-08-0 |  |
| 6(2011) | Danofloxacine |  |  |  |
| 10(2011-2014) | Dapson | Dapsone | 80-08-0 |  |
| 2(2011) | Dapsone |  |  |  |
| 8(2010–11) | Dehydro- erythromycin | Dehydro- erythromycin |  |  |
| 6(2011) | Dehydronifedipine | Oxidised nifedipine | 67035-22-7 |  |
| 8(2010–11) | Desmethyldiazepam | Desmethyldiazepam | 1088-11-5 |  |
| 7(2013) | Desmethylenlafaxine | Desmethylenlafaxine |  |  |
| 7(2013) | Desmethyltramadol | Desmethyltramadol |  |  |
| 6(2011) | Desvenlafaxine | Desvenlafaxine | 93413-62-8 |  |
| 10(2011-2014) | Diatrizoic acid | Diatrizoic acid | 737-31-5 | CAS is for sodium amidotrizoate; either name or CAS incorrect |
| 2(2011); 6(2011); 7(2013); 8(2010–11); 10(2011-2014); 13(2011-14) | Diazepam | Diazepam | 439-14-5 |  |
| 2(2009-2011); 3(2014); 4(2008); 6(2011); 7(2013); 8(2010–11); 9(2013); 10(2011-2014); 10(2012); 10(2014); 10(2005/2006); 11(2014); 12(2013) | Diclofenac | Diclofenac | 15307-86-5 |  |
| 13(2011-14) | Diclofenac artifact | Diclofenac artifact | 15362-40-0 | IUPAC Name 1-(2,6-dichlorophenyl)-1,3-dihydro-2H-indol-2-one |
| 2(2011); 10(2011-2014) | Dicloxacillin | Dicloxacillin | 3116-76-5 |  |
| 3(2014) | Dicyclanil | Dicyclanil |  |  |
| 10(2012) | Diethylenetriaminepentaacetic acid (DTPA) | Diethylenetriaminepentaacetic acid (DTPA) |  |  |
| 7(2013) | Diethylstilbestrol | Diethylstilbestrol |  |  |
| 7(2013) | Dihydrocodein | Dihydrocodeine | 125-28-0 |  |
| 6(2011) | Dihydrocodeine |  |  |  |
| 6(2011) | Dihydrostreptomycine Sulfate | Dihydrostreptomycine Sulfate | 57-92-1 |  |
| 6(2011) | Diltiazem | Diltiazem | 42399-41-7 |  |
| 2(2011) | Dimethylaminophenazone | Dimethylaminophenazone | 58-15-1 |  |
| 10(2011-2014); 13(2011-14) | Dimetridazole | Dimetridazole | 551-92-8 |  |
| 13(2011-14) | Diphenhydramine | Diphenhydramine | 58-73-1 |  |
| 6(2011) | Diosmetine | Diosmetine | 520-34-3 |  |
| 6(2011) | Dopamine | Dopamine | 51-61-6 |  |
| 7(2013) | Doxycyclin | Doxycycline | 94088-85-4 |  |
| 2(2011); 6(2011) | Doxycycline |  |  |  |
| 6(2011) | Drospirenone | Drospirenone | 67392-87-4 |  |
| 6(2011) | Dydrogesterone | Dydrogesterone | 152-62-5 |  |
| 5(2006); 8(2010–11); 10(2011-2014) | Enalapril | Enalapril | 75847-73-3 |  |
| 2(2011); 10(2011-2014) | Enoxacin | Enoxacin | 74011-58-8 |  |
| 6(2011) | Enoxacine |  |  |  |
| 1(2008); 2(2011); 5(2006); 7(2013); 10(2011-2014) | Enrofloxacin | Enrofloxacin | 93106-60-6 |  |
| 6(2011) | Enrofloxacine |  |  |  |
| 1(2008) | Epi-Tetrazyklin | Epi-Tetrazyklin |  |  |
| 3(2014) | Epoxycarbamazepine | Epoxycarbamazepine |  |  |
| 12(2013) | Erthromycin | Erythromycin | 114-07-8 |  |
| 1(2008); 2(2011); 5(2006); 8(2010–11); 7(2013); 10(2011-2014) | Erythromycin |  |  |  |
| 6(2011) | Erythromycine |  |  |  |
| 10(2005/2006); 10(2012) | erytromycine |  |  |  |
| 2(2011) | Estriol | Estriol | 50-27-1 |  |
| 2(2011); 4(2008); 6(2011); 8(2010–11); 10(2011-2014) | Estrone | Estrone | 53-16-7 |  |
| 10(2012) | oestron |  |  |  |
| 10(2012) | ethyleendiaminetetra-azijnzuur (EDTA) | Ethylenediaminetetraacetic acid (EDTA) |  | Group of substances - unclear which one. ? pharmaceutical |
| 7(2013) | Etofibrat | Etofibrate | 31637-97-5 |  |
| 2(2011) | Etofibrate |  |  |  |
| 6(2011) | Fenbendazole | Fenbendazole | 43210-67-9 |  |
| 10(2005/2006) | fenofibraat | Fenofibrate | 49562-28-9 |  |
| 7(2013) | Fenofibrat |  |  |  |
| 2(2011); 6(2011); 10(2011-2014); 11(2014) | Fenofibrate |  |  |  |
| 2(2011) | Fenofibric acid | Fenofibric acid | 2206-57-1 / 42017-89-0 | different numbers - EU-site / producers |
| 7(2013) | Fenofibrinsäure |  |  |  |
| 2(2011); 7(2013); 10(2011-2014) | Fenoprofen | Fenoprofen | 31879-05-7 |  |
| 3(2014); 5(2006); 6(2011) | Florfenicol | Florfenicol | 73231-34-2 |  |
| 2(2011) | Florfenicole |  |  |  |
| 10(2011-2014) | Flucloxacillin | Flucloxacillin | 5250-39-5 |  |
| 6(2011); 10(2012); 10(2011-2014) | Flumequine | Flumequine | 42835-25-6 |  |
| 7(2013) | Fluoxetin | Fluoxetine | 54910-89-3 | Active substance in Prozac (see Prozac – different CAS) |
| 6(2011); 13(2011-14) | Fluoxetine |  |  |  |
| 10(2011-2014) | Fluoxetine |  | 56296-78-7 |  |
| 6(2011) | Fluvoxamine | Fluvoxamine | 54739-18-3 |  |
| 7(2013) | Formoterol | Formoterol |  |  |
| 10(2012) | furazolidon | Furazolidone | 67-45-8 |  |
| 2(2011) | Furazolidone |  |  |  |
| 5(2006) | furosemid | Furosemide | 54-31-9 |  |
| 6(2011); 8(2010–11); 10(2011-2014) | Furosemide |  |  |  |
| 7(2013); 13(2011-14) | Gabapentin | Gabapentin | 60142-96-3 |  |
| 10(2012) | Galaxolide (I en II) | Galaxolide (I and II) | 1222-05-5 |  |
| 2(2011); 4(2008); 6(2011); 7(2013); 8(2010–11); 10(2012); 10(2011-2014); 11(2014); 12(2013) | Gemfibrozil | Gemfibrozil | 25812-30-0 |  |
| 6(2011) | Gestodene | Gestodene | 60282-87-3 |  |
| 13(2011-14) | Guaifenesin | Guaifenesin | 93-14-1 |  |
| 6(2011) | Heroine | Heroine | 561-27-3 |  |
| 6(2011); 8(2010–11) | Hydrochlorothiazide | Hydrochlorothiazide | 58-93-5 |  |
| 6(2011) | Hydrocodone | Hydrocodone | 125-29-1 |  |
| 6(2011) | Hydroxycarbamide | Hydroxycarbamide | 127-07-1 |  |
| 3(2014) | Hydroxyibuprofene | Hydroxyibuprofene |  |  |
| 6(2011) | Hydroxy-metronidazole | Hydroxy-metronidazole | 4812-40-2 |  |
| 2(2009-2011); 4(2008); 5(2006); 7(2013); 8(2010–11); 10(2005/2006); 10(2012); 10(2011-2014); 10(2014); 12(2013); | Ibuprofen | Ibuprofen | 15687-27-1 |  |
| 6(2011) | Ibuprofene |  |  |  |
| 7(2013) | Ifosfamid | Ifosfamide | 3778-73-2 |  |
| 2(2011); 10(2005/2006); 10(2011-2014) | Ifosfamide |  |  |  |
| 6(2011) | Imatinib | Imatinib | 152459-95-5 |  |
| 7(2013) | Indometacin | Indomethacine | 53-86-1 |  |
| 2(2011); 10(2011-2014); 11(2014) | Indomethacine |  |  |  |
| 6(2011) | Iobitridol | Iobitridol | 136949-58-1 |  |
| 7(2013) | Iodipamid | Iodipamide | 606-17-7 |  |
| 2(2011) | Iodipamide |  |  |  |
| 2(2011); 7(2013); 10(2005/2006); 10(2011-2014) | iohexol | iohexol | 66108-95-0 |  |
| 10(2012) | johexol |  |  |  |
| 2(2011); 3(2014); 7(2013); 10(2011-2014) | Iomeprol | Iomeprol | 78649-41-9 | not on ECHA |
| 10(2012) | jomeprol |  |  |  |
| 2(2011); 6(2011); 7(2013); 10(2005/2006); 10(2011-2014) | Iopamidol | Iopamidol | 62883-00-5 |  |
| 10(2012) | jopamidol |  |  |  |
| 10(2011-2014) | Iopanoic acid | Iopanoic acid | 96-83-3 |  |
| 7(2013) | Iopansäure |  |  |  |
| 7(2013); 12(2013) | Iopromid | iopromide | 73334-07-3 |  |
| 2(2011); 10(2005/2006); 10(2011-2014) | iopromide |  |  |  |
| 10(2012) | jopromide |  |  |  |
| 2(2011) | Iot(h)alamic acid | Iothalamic acid | 2276-90-6 |  |
| 7(2013) | Iot(h)alaminsäure |  |  |  |
| 10(2011-2014) | Iothalamic acid |  |  |  |
| 2(2011) | Ioxaglic acid | Ioxaglic acid | 59017-64-0 |  |
| 7(2013) | Ioxaglinsäure |  |  |  |
| 2(2011) | Ioxithalamic acid | Ioxithalamic acid | 28179-44-4 |  |
| 10(2011-2014) | Ioxithalamic acid |  |  |  |
| 7(2013) | Ioxithalaminsäure |  |  |  |
| 10(2012) | joxitalaminezuur |  |  |  |
| 3(2014) | Irbesartan | Irbesartan |  |  |
| 6(2011) | Isoquinoline | Isoquinoline | 119-65-3 |  |
| 7(2013); 10(2012) | Ivermectin | Ivermectine | 70288-86-7 (70161-11-4 + 70209-81-3) |  |
| 6(2011) | Ivermectine |  | 70288-86-7 |  |
| 2(2009-2011); 4(2008); 7(2013); 8(2010–11); 9(2013); 10(2011-2014); 10(2012); 10(2014); 11(2014); 12(2013) | Ketoprofen | Ketoprofen | 22071-15-4 |  |
| 3(2014); 6(2011) | Ketoprofene |  |  |  |
| 13(2011-14) | Lamotrigine | Lamotrigine | 84057-84-1 |  |
| 13(2011-14) | Lasalocid A-sodium | Lasalocid A-sodium | 25999-20-6 |  |
| 3(2014); 6(2011); 13(2011-14) | Levamisole | Levamisole | 14769-73-4 |  |
| 6(2011) | Levonorgestrel | Levonorgestrel | 797-63-7 |  |
| 7(2013) | Lidocain | Lidocaine | 137-58-6 |  |
| 9(2013); 10(2012); 10(2014); 10(2011-2014); 13(2011-14) | lidocaine |  |  |  |
| 7(2013); 8(2010–11); 10(2011-2014); 13(2011-14) | Lincomycin | Lincomycin | 154-21-2 |  |
| 3(2014); 10(2005/2006); 10(2012) | lincomycine |  |  |  |
| 6(2011) | Lincomycine |  | 859-18-7 |  |
| 6(2011) | Lorazepam | Lorazepam | 846-49-1 |  |
| 3(2014); 6(2011) | Losartan | Losartan | 114798-26-4 |  |
| 6(2011) | Maduramicine | Maduramicin | 84878-61-5 |  |
| 10(2011-2014) | Malachite Green | Malachite Green | 569-64-2 |  |
| 7(2013) | Malachitgrün |  |  |  |
| 1(2008) | Marbofloxacin | Marbofloxacin | 115550-35-1 |  |
| 6(2011) | Marbofloxacine |  |  |  |
| 10(2011-2014) | Mebendazole | Mebendazole | 31431-39-7 |  |
| 2(2011) | Meclocycline | Meclocycline | 2013-58-3 |  |
| 12(2013) | Mefenaminsaeure | Mefenamic Acid |  |  |
| 2(2011); 3(2014); 7(2013) | Mestranol | Mestranol | 72-33-3 |  |
| 7(2013); 10(2012) | Metformin | Metformin | 657-24-9 |  |
| 6(2011) | Metformine |  |  |  |
| 13(2011-14) | Methadone | Methadone | 76-99-3 |  |
| 13(2011-14) | Methaqualone | Methaqualone | 72-44-6 |  |
| 8(2010–11) | Methotrexate | Methotrexate | 59-05-2 |  |
| 5(2006) | methyl-triclosan | methyl-triclosan |  |  |
| 13(2011-14) | Methylprednisolone | Methylprednisolone | 83-43-2 |  |
| 2(2011); 3(2014); 6(2011); 7(2013); 10(2005/2006); 10(2011-2014); 10(2012); 10(2014); 11(2014); 12(2013) | Metoprolol | Metoprolol | 37350-58-6 |  |
| 3(2014) | Metrifonate=Trichlorfon | Metrifonate=Trichlorfon |  |  |
| 2(2011); 6(2011); 10(2011-2014); 12(2013) | Metronidazole | Metronidazole | 443-48-1 |  |
| 13(2011-14) | Miconazole | Miconazole | 22916-47-8 |  |
| 6(2011) | Monensine | Monensine | 17090-79-8 |  |
| 13(2011-14) | Modafinile | Modafinile | 68693-11-8 |  |
| 6(2011) | Morphine | Morphine | 57-27-2 |  |
| 6(2011) | N4-Acetylsulfamethoxazole | N4-Acetylsulfamethoxazole | 723-46-6 | Name not on ECHA (CAS correct?) |
| 7(2013) | N-Acetyl-4-aminoantipyrin | N-Acetyl-4-aminoantipyrin |  |  |
| 7(2013) | N-Acetyl-Sulfadiazin | N-Acetyl-Sulfadiazine |  |  |
| 7(2013) | Nadolol | Nadolol |  |  |
| 2(2011) | Nafcillin | Nafcillin | 147-52-4 |  |
| 3(2014) | Nafronyl | Nafronyl |  |  |
| 13(2011-14) | Naltrexone | Naltrexone | 16590-41-3 |  |
| 2(2011); 4(2008); 7(2013); 8(2010–11); 10(2011-2014); 10(2012); 10(2014); 12(2013) | Naproxen | Naproxen | 22204-53-1 |  |
| 3(2014); 6(2011) | Naproxene |  |  |  |
| 6(2011) | Narasine | Narasine | 55134-13-9 |  |
| 13(2011-14) | Nicotine | Nicotine | 54-11-5 |  |
| 6(2011) | Norethindrone | Norethisterone | 68-22-4 | Norethindrone not on ECHA, probably norethisterone |
| 2(2011) | Norethisteron |  |  |  |
| 2(2011); 10(2011-2014); 12(2013) | Norfloxacin | Norfloxacin | 70458-96-7 |  |
| 6(2011) | Norfloxacine |  |  |  |
| 6(2011) | Norfluoxetine | Norfluoxetine | 83891-03-6 |  |
| 6(2011) | O-Demethyltramadol | O-Demethyltramadol | 73986-53-5 |  |
| 6(2011) | o-desmethylnaproxene | o-desmethylnaproxene | 23981-47-7 |  |
| 2(2011); 8(2010–11); 10(2011-2014); 12(2013) | Ofloxacin | Ofloxacin | 82419-36-1 |  |
| 6(2011) | Ofloxacine |  |  |  |
| 6(2011) | o-Hydroxyatorvastatin | o-Hydroxyatorvastatin | 214217-86-4 |  |
| 2(2011); 10(2011-2014) | Oleandomycin | Oleandomycin | 3922-90-5 |  |
| 10(2012) | oleandomycine | Oleandomycin | 7060-74-4 | this is oleandomycin-phosphate |
| 5(2006) | omeprazol | Omeprazole | 73590-58-6 |  |
| 6(2011); 8(2010–11) | Omeprazole |  |  |  |
| 10(2011-2014); 12(2013) | Oseltamivir | Oseltamivir | 196618-13-0 |  |
| 2(2011) | Oxacillin | Oxacillin | 66-79-5 |  |
| 3(2014); 6(2011); 7(2013) | Oxazepam | Oxazepam | 604-75-1 |  |
| 7(2013) | Oxcarbazepin | Oxcarbazepine | 28721-07-5 |  |
| 13(2011-14) | Oxcarbazepine |  |  |  |
| 10(2011-2014); 13(2011-14) | Oxolinic acid | Oxolinic acid | 14698-29-4 |  |
| 6(2011) | Oxolinique Acide |  |  |  |
| 5(2006) | oxolinsyre | oxolinsyre |  |  |
| 7(2013) | Oxprenolol | Oxprenolol |  |  |
| 6(2011) | Oxycodone | Oxycodone | 76-42-6 |  |
| 10(2011-2014) | Oxymetazoline | Oxymetazoline | 1491-59-4 |  |
| 7(2013) | Oxytetracyclin | Oxytetracycline | 79-57-2 |  |
| 2(2011); 6(2011); 8(2010–11); 10(2011-2014) | Oxytetracycline |  |  |  |
| 5(2006) | oxytetracyklin |  |  |  |
| 1(2008) | Oxytetrazyklin |  |  |  |
| 2(2011); 3(2014); 5(2006); 6(2011); 7(2013); 8(2010–11); 10(2011-2014); 10(2012); 10(2014); 10(2005/2006); 11(2014); 12(2013); 13(2011-14) | Paracetamol | Paracetamol (Acetominophen) | 103-90-2 |  |
| 2(2011); 6(2011); 11(2014) | Penicillin G | Penicillin G | 61-33-6 |  |
| 2(2011) | Penicillin V | Penicillin V |  |  |
| 13(2011-14) | Pentobarbital | Pentobarbital | 76-74-4 |  |
| 2(2011); 7(2013) | Pentox(i)yfyllin | Pentoxifylline | 06-05-93 |  |
| 10(2012); 10(2011-2014) | Pentoxifylline |  |  |  |
| 6(2011) | Perindoprilat | Perindoprilat | 95153-31-4 |  |
| 2(2011); 10(2011-2014) | Phenacetin | Phenacetin | 62-44-2 |  |
| 6(2011) | Phenazine | Phenazine | 92-82-0 |  |
| 10(2005/2006); 10(2012) | Fenazon | Phenazone | 60-80-0 |  |
| 2(2011); 7(2013) | Phenazon |  |  |  |
| 6(2011); 10(2011-2014) | Phenazone |  |  |  |
| 5(2006) | phenoxymethylpenicillin | phenoxymethylpenicillin |  |  |
| 13(2011-14) | Phenylbutazone | Phenylbutazone | 50-33-9 |  |
| 6(2011) | p-Hydroxyatorvastatin | p-Hydroxyatorvastatin | 214217-88-6 |  |
| 2(2011); 7(2013); 10(2011-2014) | Pindolol | Pindolol | 13523-86-9 |  |
| 7(2013) | Piroxicam | Piroxicam |  |  |
| 6(2011) | Pravastatine | Pravastatine | 81131-70-6 |  |
| 6(2011) | Prednisolone | Prednisolone | 50-24-8 |  |
| 2(2011); 7(2013); 12(2013) | Primidon | Primidon | 125-33-7 |  |
| 10(2011-2014); 13(2011-14) | Primidone |  |  |  |
| 3(2014); 6(2011) | Progesterone | Progesterone | 57-83-0 |  |
| 13(2011-14) | Propofol | Propofol | 2078-54-8 |  |
| 2(2011); 6(2011); 10(2011-2014); 11(2014); 12(2013); 13(2011-14) | Propranolol | propranolol | 525-66-6 |  |
| 7(2013) | Propranolol (=Propanolol) |  |  |  |
| 10(2014) | Propranolol |  | 318-98-9 |  |
| 2(2011); 7(2013) | Propyphenazon | Propyphenazon | 479-92-5 |  |
| 10(2011-2014) | Propyphenazone |  |  |  |
| 10(2005/2006) | prozac | prozac | 59333-67-4 | See fluoxetine |
| 3(2014); 6(2011) | Ramipril | Ramipril | 87333-19-5 |  |
| 3(2014); 6(2011) | Ramiprilate | Ramiprilate | 87269-97-4 |  |
| 3(2014); 6(2011); 8(2010–11); 10(2011-2014); | Ranitidine | Ranitidine | 66357-35-5 |  |
| 2(2011); 10(2011-2014) | Ronidazole | Ronidazole | 7681-76-7 |  |
| 3(2014) | Rosuvastatin | Rosuvastatin |  |  |
| 1(2008); 10(2011-2014) | Roxithromycin | Roxithromycin | 80214-83-1 |  |
| 10(2012) | roxitromycine |  |  |  |
| 2(2011); 7(2013) | Roxythromycin |  |  |  |
| 2(2011); 7(2013); 8(2010–11); 10(2011-2014) | Salbutamol | Salbutamol | 18559-94-9 |  |
| 5(2006) | salicylsyre | Salicylic acid | 69-72-7 |  |
| 10(2005/2006) | salicylzuur |  |  |  |
| 6(2011) | Sertraline | Sertraline | 79617-96-2 |  |
| 8(2010–11) | Sildenafil | Sildenafil | 171599-83-0 |  |
| 2(2011); 12(2013) | Simvastatin | Simvastatin | 79902-63-9 |  |
| 6(2011) | Simvastatine |  |  |  |
| 2(2011); 3(2014); 5(2006); 6(2011); 7(2013); 10(2011-2014); 10(2012); 10(2014); 10(2005/2006); 12(2013); 13(2011-14) | Sotalol | Sotalol | 3930-20-9 |  |
| 2(2011); 8(2010–11); 10(2011-2014) | Spiramycin | Spiramycin | 8025-81-8 |  |
| 10(2012) | sulfachloorpyridazine | Sulfachloropyridazin | 80-32-0 |  |
| 7(2013) | Sulfachloropyridazin |  |  |  |
| 10(2011-2014) | Sulfachloropyridazine |  |  |  |
| 7(2013) | Sulfachlorpyridazin | Sulfachlorpyridazin |  |  |
| 1(2008); 5(2006); 7(2013); 12(2013) | Sulfadiazin | Sulfadiazine | 68-35-9 |  |
| 2(2011); 3(2014); 6(2011); 10(2012); 10(2011-2014); 13(2011-14) | Sulfadiazine |  |  |  |
| 7(2013) | Sulfadiazin 4-OH | Sulfadiazin 4-OH |  |  |
| 7(2013) | Sulfadiazin N-Ac | Sulfadiazin N-Ac |  |  |
| 7(2013); 12(2013) | Sulfadimethoxin | Sulfadimethoxine | 122-11-2 |  |
| 6(2011); 7(2013); 10(2011-2014) | Sulfadimethoxine |  |  |  |
| 1(2008); 5(2006); 7(2013) | Sulfadimidin | Sulfadimidine | 57-68-1 | also known as sulfamethazine |
| 2(2011); 10(2012) | sulfadimidine |  |  |  |
| 3(2014); 6(2011); 10(2011-2014); 11(2014); 13(2011-14) | Sulfamethazine |  |  |  |
| 7(2013); 12(2013) | Sulfamethazin |  |  |  |
| 1(2008); 2(2011); 5(2006); 7(2013) | Sulfadoxin | Sulfadoxin |  |  |
| 7(2013) | Sulfaethoxypyridazin | Sulfaethoxypyridazin |  |  |
| 7(2013) | Sulfamerazin | Sulfamerazine | 127-79-7 |  |
| 2(2011); 10(2011-2014) | Sulfamerazine |  |  |  |
| 5(2006) | sulfamethizol | Sulfamethizole | 144-82-1 |  |
| 13(2011-14) | Sulfamethizole |  |  |  |
| 1(2008); 5(2006); 7(2013); 10(2005-2006); 10(2012); 12(2013) | Sulfamethoxazol | Sulfamethoxazole | 723-46-6 |  |
| 2(2009-2011); 3(2014); 4(2008); 6(2011); 8(2010–11); 10(2011-2014); 11(2014); 13(2011-14) | Sulfamethoxazole |  |  |  |
| 10(2014) | sulfametoxazool |  |  |  |
| 10(2005/2006) | sulphamethoxazol |  |  |  |
| 2(2011) | Sulfamethoxine | Sulfamethoxine | 651-06-9 | see sulphisoxazole. ECHA name for this CAS is Sulfametoxydiazine, Sulfamethoxine and Sulfisoxazole not in ECHA |
| 2(2011) | Sulfisoxazole | Sulfisoxazole | 651-06-9 |  |
| 7(2013) | Sulfamethoxypyridazin | Sulfamethoxypyridazin |  |  |
| 13(2011-14) | Sulfanilamide | Sulfanilamide | 63-74-1 |  |
| 7(2013); 12(2013) | Sulfapyridin | Sulfapyridine | 144-83-2 |  |
| 10(2011-2014) | Sulfapyridine |  |  |  |
| 6(2011) | Sulfaquinoxaline | Sulfaquinoxaline | 59-40-5 |  |
| 10(2011-2014) | Sulfachinoxalin |  |  |  |
| 1(2008); 12(2013) | Sulfathiazol | Sulfathiazole | 72-14-0 |  |
| 7(2013) | Sulfathiazol [e] |  |  |  |
| 6(2011) | Sulfathiazole |  |  |  |
| 5(2006) | sulfatroxazol | sulfatroxazol |  |  |
| 8(2010–11) | Tamoxifen | Tamoxifen | 10540-29-1 |  |
| 7(2013) | Temazepam | Temazepam |  |  |
| 7(2013); 10(2011-2014) | Terbutalin | Terbutalin | 23031-25-6 |  |
| 2(2011) | Terbutaline |  |  |  |
| 7(2013) | Testosteron | Testosterone | 58-22-0 |  |
| 6(2011); 11(2014) | Testosterone |  |  |  |
| 7(2013) | Tetracyclin | Tetracycline | 60-54-8 |  |
| 2(2011); 6(2011); 10(2011-2014) | Tetracycline |  |  |  |
| 5(2006) | tetracyklin |  |  |  |
| 1(2008) | Tetrazyklin |  |  |  |
| 11(2014); 13(2011-14) | Theophylline | Theophylline | 58-55-9 |  |
| 2(2011); 5(2006) | tiamulin | Tiamulin | 55297-95-5 |  |
| 10(2011-2014) | Tiamuline |  |  |  |
| 7(2013) | Tilmicosin | Tilmicosin |  |  |
| 7(2013) | Timolol | Timolol |  |  |
| 3(2014); 6(2011); 7(2013); 13(2011-14) | Tramadol | Tramadol | 27203-92-5 |  |
| 5(2006) | triclosan | triclosan |  | ? pharmaceutical |
| 3(2014); 6(2011) | Trimetazidine | Trimetazidine | 5011-34-7 |  |
| 1(2008); 2(2011); 5(2006); 7(2013); 10(2012); 10(2011-2014); 11(2014); 12(2013); 13(2011-14) | trimethoprim | Trimethoprim | 738-70-5 |  |
| 3(2014); 6(2011) | Trimethoprime |  |  |  |
| 7(2013) | Tulathromycin | Tulathromycin |  |  |
| 2(2011); 5(2006); 10(2011-2014) | Tylosin | Tylosin | 1401-69-0 |  |
| 6(2011) | Tylosine |  |  |  |
| 8(2010–11) | Vancomycin | Vancomycin | 1404-90-6 |  |
| 12(2013) | Venlafaxin | Venlafaxine |  |  |
| 2(2011); 3(2014); 7(2013) | Venlafaxine | Venlafaxine | 99300-78-4 | for salt - not clear if same meant |
| 7(2013) | Verapamil | Verapamil |  |  |
| 2(2011) | Virginiamycin | Virginiamycin | 11006-76-1 |  |
| 6(2011) | Zolpidem | Zolpidem | 82626-48-0 |  |
| 10(2012) | β-apo-oxytetracycline | β-apo-oxytetracycline | 18751-99-0 |  |

# Annex 3 : Aggregated data - Master Table

(the electronic file is included in Annex 4)

| Substance (Common Name) | No. of PC | Total no of sites | Aggregated data from all PC datasets |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | >LoQ | LoQ to 0,05 µg/l | 0.05 - 0.1 µg/l | 0.1 - 1.0 µg/l | 1.0 - 3.0 µg/l | 3.0 - 10.0 µg/l | > 10 µg/l | Detects >0.1 µg/l |
| 14-Hydroxyclarithromycin | 1 | 464 | 0 |  |  |  |  |  |  |  |
| 17-alpha-Estradiol | 3 | 722 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1-Hydroxy Ibuprofen | 1 | 488 | 3 | 3 |  |  |  |  |  |  |
| 2-hydroxy-ibuprofen | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-Acetamidoantipyrin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-acetylaminoantipyrine | 1 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-Chlorobenzoic acid | 1 | 492 | 1 |  |  | 1 |  |  |  |  |
| 4-epi-Chlortetracyclin | 2 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-epi-Oxytetracyclin | 1 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-epi-Tetracyclin | 1 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4-Formylaminoantipyrin | 2 | 137 | 7 | 4 | 3 | 0 | 0 | 0 | 0 | 0 |
| 4-Hydroxy-Sulfadiazin | 1 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Acetbutolol | 1 | 159 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Acetylsalicylic acid | 3 | 600 | 7 | 0 | 0 | 7 | 0 | 0 | 0 | 7 |
| Acetyl-Sulfadimidine | 1 | 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Acetyl-sulfamethoxazole | 2 | 243 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Albendazole | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Alprazolam | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Altrenogest | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Amidotrizoate | 1 | 24 | 5 | 0 | 3 | 2 | 0 | 0 | 0 | 2 |
| Amiodarone | 1 | 492 | 5 | 5 |  |  |  |  |  |  |
| Amlodipine | 2 | 495 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Amoxicillin | 4 | 701 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ampicillin | 2 | 495 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Amprolium | 1 | 492 | 1 | 1 |  |  |  |  |  |  |
| Androstadienedione | 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| Androstenedione | 1 | 492 | 1 |  |  | 1 |  |  |  |  |
| Androsterone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Anhydro erythromycin | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Atenolol | 7 | 1499 | 10 | 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| Atorvastatin | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Azithromycin | 2 | 284 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Be(n)zafibrat | 1 | 844 | 3 | 2 | 1 | 0 | 0 | 0 | 0 | 0 |
| bendroflumethiazid | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Benzotriazole | 2 | 115 | 4 | 2 | 0 | 1 | 1 | 0 | 0 | 2 |
| benzylpenicillin | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| beta-Sitosterol | 1 | 425 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Betaxolol | 4 | 341 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bezafibrate | 7 | 884 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bisoprolol | 3 | 747 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Bisoprolol-A | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bisphenol-A | 1 | 105 | 12 | 11 | 0 | 1 | 0 | 0 | 0 | 1 |
| Bromazepam | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Bromhexin | 1 | 105 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Buflomedil | 1 | 492 | 1 | 1 |  |  |  |  |  |  |
| Butabarbital | 1 | 6 | 6 | 3 | 2 |  | 1 |  |  |  |
| Butalbital | 1 | 448 | 0 |  |  |  |  |  |  |  |
| caffeine | 2 | 490 | 252 | 133 | 60 | 53 | 5 | 0 | 1 | 59 |
| Capecitabine | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Carazolol | 1 | 158 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Carbamazepine | 12 | 3692 | 445 | 353 | 35 | 35 | 13 | 5 | 4 | 57 |
| Carbamazepine epoxide | 1 | 492 | 5 | 5 |  |  |  |  |  |  |
| Carboxyibuprofen | 1 | 492 | 2 |  |  | 2 |  |  |  |  |
| Cefazoline | 2 | 641 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cefotaxim | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cefuroxime | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Chloramphenicol | 2 | 264 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Chlormadinone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Chlorotetracycline | 2 | 154 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Chlortetracycline | 4 | 694 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| cimetidine | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ciprofloxacin | 6 | 880 | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Clarithromycin | 7 | 1216 | 10 | 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| Clenbuterol | 3 | 424 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Clindamycin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Clofibrate | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Clofibric acid | 7 | 2294 | 46 | 26 | 10 | 10 | 0 | 0 | 0 | 10 |
| Clonazepam | 1 | 2 | 2 | 2 |  |  |  |  |  |  |
| Clopidol | 1 | 11 | 11 | 11 |  |  |  |  |  |  |
| Clorsulone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Clotrimazole | 2 | 917 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cloxacillin | 3 | 646 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cocaine | 2 | 493 | 2 | 1 |  | 1 |  |  |  |  |
| Codeine | 4 | 666 | 15 | 15 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cotinine | 1 | 1 | 1 |  |  | 1 |  |  |  |  |
| Crotamiton | 1 | 27 | 27 | 18 | 4 | 3 | 1 | 1 |  |  |
| Cyamemazine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Cyclophosphamide | 5 | 958 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Cytarabine | 1 | 492 | 2 | 2 |  |  |  |  |  |  |
| Danofloxacin | 2 | 542 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Dapsone | 2 | 154 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Dehydro- erythromycin | 1 | 55 | 7 | 7 | 0 | 0 | 0 | 0 | 0 | 0 |
| Desmethyldiazepam | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Desmethylenlafaxine | 1 | 129 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Desmethyltramadol | 1 | 129 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Desvenlafaxine | 1 | 492 | 5 | 4 | 1 |  |  |  |  |  |
| Diatrizoic Acid | 5 | 1394 | 66 | 40 | 11 | 14 | 1 | 0 | 0 | 15 |
| Diazepam | 6 | 879 | 9 | 9 | 0 | 0 | 0 | 0 | 0 | 0 |
| Diclofenac | 10 | 3060 | 34 | 14 | 7 | 13 | 0 | 0 | 0 | 13 |
| Diclofenac artifact | 1 | 1 | 1 |  |  | 1 |  |  |  |  |
| Dicloxacillin | 2 | 154 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Dicyclanil | 1 | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Diethylenetriaminepentaacetic acid (DTPA) | 1 | 105 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| Diethylstilbestrol | 1 | 125 | 0 |  |  |  |  |  |  |  |
| Dihydrocodeine | 2 | 493 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Dihydrostreptomycine Sulfate | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Diltiazem | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Dimethylaminophenazone | 1 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Dimetridazole | 2 | 150 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| Diosmetine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Dopamine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Doxycycline | 3 | 636 | 3 | 2 | 1 | 0 | 0 | 0 | 0 | 0 |
| Drospirenone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Dydrogesterone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Enalapril | 3 | 207 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Enoxacin | 3 | 646 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Enrofloxacin | 6 | 858 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Epi-Tetrazyklin | 1 | 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Epoxycarbamazepine | 1 | 32 | 6 | 6 | 0 | 0 | 0 | 0 | 0 | 0 |
| Erythromycin | 8 | 1255 | 25 | 18 | 4 | 2 | 1 | 0 | 0 | 3 |
| Estradiol | 4 | 683 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Estriol | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Estrone | 5 | 818 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ethinyl estradiol | 5 | 793 | 5 | 5 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ethylenediaminetetraacetic acid (EDTA) | 1 | 105 | 61 | 0 | 0 | 0 | 16 | 26 | 19 | 61 |
| Etofibrate | 2 | 76 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Fenbendazole | 1 | 492 | 1 | 1 |  |  |  |  |  |  |
| Fenofibrate | 5 | 746 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Fenofibric acid | 4 | 601 | 14 | 11 | 1 | 2 | 0 | 0 | 0 | 2 |
| Fenoprofen | 3 | 457 | 5 | 0 | 5 | 0 | 0 | 0 | 0 | 0 |
| Florfenicol | 4 | 533 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Flucloxacillin | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Flumequine | 2 | 746 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Fluoxetine | 3 | 877 | 2 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| Fluvoxamine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Formoterol | 1 | 159 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Furazolidone | 2 | 110 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Furosemide | 4 | 699 | 6 | 5 | 0 | 1 | 0 | 0 | 0 | 1 |
| Gabapentin | 2 | 167 | 19 | 2 | 4 | 10 | 1 | 0 | 2 | 13 |
| Galaxolide (I and II) | 1 | 105 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Gemfibrozil | 8 | 1085 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Gestodene | 1 | 492 | 1 | 1 |  |  |  |  |  |  |
| Heroine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Hydrochlorothiazide | 2 | 547 | 12 | 11 | 0 | 0 | 1 | 0 | 0 | 1 |
| Hydrocodone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Hydroxycarbamide | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Hydroxyibuprofene | 1 | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Hydroxy-metronidazole | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Ibuprofen | 9 | 1865 | 49 | 26 | 8 | 8 | 2 | 2 | 3 | 15 |
| Ifosfamide | 3 | 287 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Imatinib | 1 | 492 | 5 | 3 | 1 | 1 |  |  |  |  |
| Indomethacine | 4 | 553 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Iobitridol | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Iodipamide | 2 | 75 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| iohexol | 3 | 630 | 2 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| Iomeprol | 4 | 757 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| Iopamidol | 4 | 1462 | 37 | 20 | 14 | 3 | 0 | 0 | 0 | 3 |
| Iopanoic acid | 2 | 308 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| iopromide | 4 | 781 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Iothalamic acid | 3 | 383 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ioxaglic acid | 2 | 75 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ioxithalamic acid | 3 | 329 | 12 | 1 | 7 | 4 | 0 | 0 | 0 | 4 |
| Irbesartan | 1 | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Isoquinoline | 1 | 492 | 22 | 22 |  |  |  |  |  |  |
| Ivermectine | 3 | 726 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| Ketoprofen | 10 | 1275 | 8 | 6 | 0 | 1 | 1 | 0 | 0 | 2 |
| Lamotrigine | 1 | 3 | 3 | 3 |  |  |  |  |  |  |
| Levamisole | 2 | 524 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Levonorgestrel | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Lidocaine | 4 | 540 | 11 | 8 | 1 | 1 | 0 | 0 | 1 | 2 |
| Lincomycin | 5 | 867 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Lorazepam | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Losartan | 2 | 524 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Maduramicin | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Malachite Green | 2 | 278 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Marbofloxacin | 2 | 542 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Mebendazole | 1 | 149 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Meclocycline | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Mefenamic Acid | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Mestranol | 3 | 162 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Metformin | 3 | 787 | 56 | 41 | 3 | 12 | 0 | 0 | 0 | 12 |
| Methaqualone | 1 | 3 | 3 | 2 |  | 1 |  |  |  |  |
| Methotrexate | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| methyl-triclosan | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Metoprolol | 7 | 1708 | 3 | 2 | 1 | 0 | 0 | 0 | 0 | 0 |
| Metrifonate=Trichlorfon | 1 | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Metronidazole | 4 | 676 | 16 | 11 | 3 | 2 | 0 | 0 | 0 | 2 |
| Monensine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Morphine | 1 | 492 | 1 | 1 |  |  |  |  |  |  |
| N4-Acetylsulfamethoxazole | 1 | 492 | 0 |  |  |  |  |  |  |  |
| N-Acetyl-4-aminoantipyrin | 1 | 129 | 5 | 3 | 1 | 1 | 0 | 0 | 0 | 1 |
| N-Acetyl-Sulfadiazine | 1 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| N-Acetyl-Sulfamethoxazole | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nadolol | 1 | 160 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nafcillin | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Nafronyl | 1 | 32 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Naproxen | 8 | 1374 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| Narasine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Nicotine | 1 | 2 | 2 | 2 |  |  |  |  |  |  |
| Norethisterone | 2 | 497 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Norfloxacin | 4 | 676 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Norfluoxetine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| O-Demethyltramadol | 1 | 492 | 10 | 10 |  |  |  |  |  |  |
| o-desmethylnaproxene | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Ofloxacin | 5 | 731 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| o-Hydroxyatorvastatin | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Oleandomycin | 2 | 259 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Omeprazole | 3 | 550 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oseltamivir | 2 | 150 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxacillin | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxazepam | 3 | 525 | 29 | 29 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxcarbazepine | 2 | 426 | 6 | 5 | 0 | 1 | 0 | 0 | 0 | 1 |
| Oxidised nifedipine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Oxolinic acid | 2 | 641 | 7 | 4 | 1 | 2 | 0 | 0 | 0 | 2 |
| oxolinsyre | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxprenolol | 1 | 159 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxycodone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Oxymetazoline | 1 | 149 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Oxytetracycline | 7 | 903 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Paracetamol (Acetominophen) | 9 | 1036 | 248 | 215 | 22 | 10 | 1 | 0 | 0 | 11 |
| Penicillin G | 3 | 521 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Penicillin V | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Pentobarbital | 1 | 26 | 26 | 6 | 4 | 13 | 3 |  |  |  |
| Pentoxifylline | 3 | 859 | 9 | 0 | 0 | 9 | 0 | 0 | 0 | 9 |
| Perindoprilat | 1 | 448 | 0 |  |  |  |  |  |  |  |
| Phenacetin | 2 | 157 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Phenazine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Phenazone | 4 | 1477 | 40 | 25 | 10 | 4 | 1 | 0 | 0 | 5 |
| phenoxymethylpenicillin | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Phenylbutazone | 1 | 1 | 1 | 1 |  |  |  |  |  |  |
| p-Hydroxyatorvastatin | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Pindolol | 3 | 316 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Piroxicam | 1 | 107 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Pravastatine | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Prednisolone | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Primidon | 4 | 628 | 31 | 16 | 5 | 10 | 0 | 0 | 0 | 10 |
| Progesterone | 2 | 524 | 3 | 2 | 0 | 1 | 0 | 0 | 0 | 1 |
| Propofol | 1 | 2 | 2 | 1 |  |  | 1 |  |  |  |
| propranolol | 6 | 1177 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Propyphenazon | 3 | 954 | 11 | 6 | 3 | 1 | 1 | 0 | 0 | 2 |
| prozac | 1 | 5 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ramipril | 2 | 524 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ramiprilate | 2 | 524 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ranitidine | 4 | 728 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ronidazole | 2 | 154 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Rosuvastatin | 1 | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Roxithromycin | 4 | 338 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Salbutamol | 4 | 478 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Salicylic acid | 2 | 8 | 5 | 4 | 1 | 0 | 0 | 0 | 0 | 0 |
| Sertraline | 1 | 492 | 0 |  |  |  |  |  |  |  |
| Sildenafil | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Simvastatin | 3 | 532 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sotalol | 6 | 1479 | 25 | 25 | 0 | 0 | 0 | 0 | 0 | 0 |
| Spiramycin | 3 | 209 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfachloropyridazin | 2 | 274 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfachlorpyridazin | 1 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfadiazin 4-OH | 1 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfadiazin N-Ac | 1 | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfadiazine | 9 | 1265 | 9 | 7 | 0 | 2 | 0 | 0 | 0 | 2 |
| Sulfadimethoxine | 4 | 702 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfadimidine | 9 | 1314 | 19 | 19 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfadoxin | 4 | 90 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfaethoxypyridazin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfamerazine | 4 | 215 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfamethizole | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfamethoxazole | 12 | 2153 | 105 | 100 | 1 | 4 | 0 | 0 | 0 | 4 |
| Sulfamethoxine | 1 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfamethoxypyridazin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfanilamide | 1 | 5 | 5 | 5 |  |  |  |  |  |  |
| Sulfapyridine | 3 | 308 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfaquinoxaline | 2 | 641 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfathiazole | 4 | 676 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| sulfatroxazol | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sulfisoxazole | 1 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tamoxifen | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Temazepam | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Terbutalin | 3 | 412 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Testosterone | 3 | 641 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tetracycline | 6 | 931 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Theophylline | 1 | 24 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tiamulin | 3 | 158 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tilmicosin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Timolol | 1 | 159 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tramadol | 3 | 675 | 54 | 51 | 2 | 1 | 0 | 0 | 0 | 1 |
| triclosan | 1 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Trimetazidine | 2 | 523 | 10 | 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| Trimethoprim | 9 | 1352 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| Tulathromycin | 1 | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tylosin | 4 | 649 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Vancomycin | 1 | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Venlafaxine | 4 | 169 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Verapamil | 1 | 129 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Virginiamycin | 1 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Zolpidem | 1 | 492 | 0 |  |  |  |  |  |  |  |
| β-apo-oxytetracycline | 1 | 105 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PC 13 unquantified detects not noted above |  |  |  |  |  |  |  |  |  |  |
| Amino-mebendazole | 1 |  |  |  |  |  |  |  |  |  |
| Amitriptyline | 1 |  |  |  |  |  |  |  |  |  |
| Celiprolol | 1 |  |  |  |  |  |  |  |  |  |
| Citalopram | 1 |  |  |  |  |  |  |  |  |  |
| Diphenhydramine | 1 |  |  |  |  |  |  |  |  |  |
| Guaifenesin | 1 |  |  |  |  |  |  |  |  |  |
| Lasalocid A-sodium | 1 |  |  |  |  |  |  |  |  |  |
| Levamisole | 1 |  |  |  |  |  |  |  |  |  |
| Methadone | 1 |  |  |  |  |  |  |  |  |  |
| Methylprednisolone | 1 |  |  |  |  |  |  |  |  |  |
| Miconazole | 1 |  |  |  |  |  |  |  |  |  |
| Modafinile | 1 |  |  |  |  |  |  |  |  |  |
| Naltrexone | 1 |  |  |  |  |  |  |  |  |  |

# Annex 4 – Other Aggregated data

HOW TO OBTAIN EU PUBLICATIONS

Free publications:

•	one copy:
via EU Bookshop (http://bookshop.europa.eu);

•	more than one copy or posters/maps:
from the European Union’s representations (http://ec.europa.eu/represent_en.htm); 
from the delegations in non-EU countries (http://eeas.europa.eu/delegations/index_en.htm); 
by contacting the Europe Direct service (http://europa.eu/europedirect/index_en.htm) or calling 00 800 6 7 8 9 10 11 (freephone number from anywhere in the EU) (*).

(*)	The information given is free, as are most calls (though some operators, phone boxes or hotels may charge you).

Priced publications:

•	via EU Bookshop (http://bookshop.europa.eu).

Priced subscriptions:

•	via one of the sales agents of the Publications Office of the European Union (http://publications.europa.eu/others/agents/index_en.htm).

[Catalogue number]

Annex 1: Nitrates; Active substances in pesticides, including their relevant metabolites, degradation and reaction products.   Annex 2: Arsenic, Cadmium, Lead, Mercury, Ammonium, Chloride, Sulphate, Trichloroethylene, Tetrachloroethylene, Conductivity, Nitrites and Phosphorus (total)/Phosphates.

amending Annex II to Directive 2006/118/EC of the European Parliament and of the Council on the protection of groundwater against pollution and deterioration

Directive 2000/60/EC of the European Parliament and of the Council of 23 October 2000 establishing a framework for Community action in the field of water policy (OJ L 327, 22.12.2000, p. 1).

Directive 2009/90/EC of 31 July 2009 laying down, pursuant to Directive 2000/60/EC of the European Parliament and of the Council, technical specifications for chemical analysis and monitoring of water status.

Quality Assurance Associates, 1999 : “Understanding Laboratory Reporting Limits” Environmental Protection Magazine, 10, 37-41

https://www.cas.org/content/chemical-substances/faqs